Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	ELECTRONIC INSTRUMENTATION				              Time : 3 hours

Subject Code: 	EC261			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	A (0 – 10) A ammeter has a guaranteed accuracy of 1% of the full scale deflection. The limiting error while reading 2.5A is _________%.

2.	Frequency can be measured by using a _________bridge.

3.	Thermistors have _________temperature coefficient of resistance.

4.	_________can act as an inverse transducer.

5.	A triangular wave is obtained by ________a square wave.

6.	Pulse rise time is defined as the time taken by the pulse to go from _______of its amplitude.

7.	Aquadag is used in CRO to collect ___________.

8.	________record one quantity with respect to another quantity.

9.	In a Digital frequency meter, __________converts sinusoidal waveforms into rectangular pulse.

10.	________is the fastest A/D converter.

PART – B  (5 x 3 = 15 MARKS)

11.	Mention the applications of a wheatsone bridge.

12.	State Piezo electric effect.

13.	Define Total Harmonic Distortion.

14.	What are the requirements of an automatic test system?

15.	State working principle of a sampling oscilloscope.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the various classifications of error with suitable examples.

(OR)

17.	Explain the working of a wheatstone bridge and derive its balance condition. Also derive an expression for current through galvanometer in case of unbalanced wheatstone bridge.

18.	Explain working principle of a strain gauge. Derive an expression for gauge factor.

(OR)

19.	Explain the construction and working of a linear variable Differential Transformer. List out the applications of LVDT.

20.	Discuss the working of a function generator with a neat block diagram.

(OR)

21.	a.	Explain the applications of a spectrum analyzer.								(8)

b.	Explain the working of a wave analyzer with a neat block diagram.				(7)

22.	Discuss the construction and working of a general purpose cathode ray oscilloscope.

(OR)

23.	a.	Mention advantages and drawbacks of FM recording.							(5)

b.	Draw and explain the block diagram of a X – Y recorder						(10)

24.	Discuss the working principle of ramp type digital voltmeter.

(OR)

25.	Write short notes on computer controlled testing of an audio amplifier and a radio receiver.