Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	TELEVISION AND VIDEO ENGINEERING			              Time : 3 hours

Subject Code: 	EC265			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Eye can store light for about ___________ of a second.

2.	Why is an aluminized coating provided on the phosphor screen of picture tube?

3.	For equal signal radiation in all directions, what type of radiation pattern is needed?

4.	Why is it necessary to prevent any deviation of IF frequencies in a colour receiver?

5.	What is the colour TV standard adopted by India?

6.	State GRASSMAN’s law.

7.	What is the name of the circuit that block the chroma channel during B&W reception in colour receiver?

8.	Obtain (G-Y) from (R-Y) and (B-Y).

9.	Why are up-link and down-link frequencies kept fairly apart from each other?

10.	How is a Camcorder different from VCR?

PART – B  (5 x 3 = 15 MARKS)

11.	What is meant deflection angle of the picture tube? What is the advantage of providing a large deflection angle yoke?

12.	Justify the choice of negative modulation for TV transmission.

13.	What do you understand by compatibility between monochrome and colour systems? How is this achieved?

14.	Why is delay line needed in Y channel?

15.	What are the aims of HDTV?

PART – C  (5 x 15 = 75 MARKS)

16.	What do you understand by interlaced scanning? Show that it reduces flicker and conserves bandwidth. With diagrams explain nominal, active and retrace intervals of horizontal and vertical scanning in 625 line system.

(OR)

17.	Sketch composite video signal of three scanning lines. Why are the front porch and back porch intervals provided?

18.	What types of modulations are used for video signal and sound signal? Why? How does the total channel width of 11.25 MHz of picture signal is reduced to 7 MHz?

(OR)

19.	Draw the block diagram of a monochrome TV receiver and briefly review the nature of input and output signals at various sections.

[P.T.O]

20.	Give answers for the following:

a.	Why tint control needed in NTSC receiver?   		 	 						(5)

b.	How differential phase errors are cancelled in PAL system?						(5)

c.	Why SECAM receiver does not need hue and saturation controls?					(5)

(OR)

21.	Draw the block diagram of a PAL encoder and explain how the composite colour signal is formed.

22.	Write notes on the following.

a.	Burst Phase Discriminator														(5)

b.	ACC amplifier																(5)

c.	Subcarrier Oscillator 															(5)

(OR)

23.	With diagram explain how U & V signals are obtained from composite video signal.

24.	With block diagram explain the functions of various VLSI chips used in digital TV receiver. What are the merits of digital TV receiver?

(OR)

25.	What is DVD stands for? How is a DVD different from a CD? Explain how a DVD carries more information than a CD.