Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	TRANSMISSION LINES AND WAVE GUIDES		               Time : 3 hours

Subject Code: 	EC280			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are distributed parameters?

2.	What do you mean by loading of transmission line?

3.	Define standing wave ratio.

4.	List the applications of smith chart.

5.	What is the cut off frequency of TEM wave?

6.	Define group velocity.

7.	How do you account for the finite conductor forming waveguide walls?

8.	What is the cut off wavelength of the dominant mode in a rectangular waveguide?

9.	Which is the dominant mode in a circular waveguide?

10.	Define Q of a cavity resonator.

PART – B  (5 x 3 = 15 MARKS)

11.	Calculate the characteristic impedance for the following line with parameters. R = 10.4 /Km,

G = 0.8 x 10 -6   /km C = 0.00835 F/km, L 0.00367 H/km at 1 KHz.

12.	A 50 line is terminated in load ZR = 75 + j 75. Determine VSWR due to this load.

13.	What are TE, TM and TEM waves?

14.	What is meant by dominant mode? Mention the dominant mode of a rectangular waveguide.

15.	What is the distinctive characteristics of the attenuation constant TEom modes in a circular waveguide?

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the general solution of a transmission line and explain its physical significance.

(OR)

17.	a.	Derive the expression for the input impedance of a transmission line.				(10)

b.	A transmission line has Z0 = 745 -12 and is terminated in ZR = 100. Calculate the

reflection loss and return loss in dB.											(5)

18.	a.	Derive the condition for dissipation less line.										(10)

b.	A transmission line is terminated in ZL. The standing wave minima are 102 cm apart and the

last minimum is 35cm from the load end of the line. The value of standing wave ratio is 2.4 and RO = 250. Determine wavelength and ZL.									(5)

(OR)

19.	a.	Deduce the expression for constant S circle for the dissipation less line.				(7)

b.	Determine the length and location of a single short circuited stub to produce and impedance

match on a transmission line with RO of 600 and terminated in 1800.			(8)

[P.T.O]

20.	a.	Discuss the characteristics of TM waves between parallel plates.					(10)

b.	For a frequency of 5GHz and plate separation of 8cm in air, find the cut off frequency, and

phase constant for TM10 mode.													(5)

(OR)

21.	a.	List the properties of TEM wave.												(5)

b.	A parallel plate waveguide has plate separation of 1cm and filled with a perfect dielectric

having dielectric constant 9. Find the cut off frequency of the dominant mode. Also calculate the phase shift constant, wavelength and phase velocity for the dominant mode for a frequency 1 KHz above fc.													(10)

22.	A rectangular air filled copper waveguide with dimensions 0.9” x 0.4” cross section and 12” length is operated at 9.2 GHz with dominant mode. Find cut off frequency, guide wavelength, phase velocity, characteristic impedance and conductor loss in dB. ( of Cu 5.8 x 107  /m)

(OR)

23.	a.	Prove that TEM wave does not exist in hollow waveguides.						(7)

b.	Establish that the wave impedance of propagating TE modes in a waveguide with a lossless

dielectric is purely resistive and is always larger than the intrinsic impedance of the dielectric medium.																		(8)

24.	An air filled circular waveguide having on inner radius 1cm is excited in dominant mode at 10GHz. Find the cut off frequency, guide wavelength, wave impedance, frequency bandwidth for operation in dominant mode only and attenuation constant for wall surface resistance of 20 m.

h11'=1.841a		;	h01=2.405a

(OR)

25.	Find an expression for unloaded Q of a rectangular cavity (a x b x a), a > b excited in dominant TE101 mode.