Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	ANTENNAS AND WAVE PROPAGATION			              Time : 3 hours

Subject Code: 	EC282			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The unit of Magnetic vector potential is ___________.

2.	The capacitance hat is used to increase _________of low frequency antenna.

3.	The relationship between effective aperture and directivity is ____________.

4.	When pitch angle  = 0º, helix becomes __________.

5.	__________is a frequency independent antenna.

6.	By using more directors in yagi-uda antenna, _______is increased.

7.	The dipole antenna is complementary to _________.

8.	The material used for dielectric lens construction is _________.

9.	Ionospheric wave propagation occurs when the frequency lies between __________.

10.	Refractive index,  = __________.

PART – B  (5 x 3 = 15 MARKS)

11.	What is retarded current?

12.	Compare directive gain and power gain.

13.	Why log periodic antenna is named so far?

14.	Find the terminal impedance of a thin λ2slot antenna when the impedance of a thin λ2dipole antenna is 73 + j 42.5 ohms.

15.	Define critical frequency of an ionized layer of ionosphere.

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the fields radiated from hertizian dipole in the near field and far field regions.

(OR)

17.	a.	Derive the expressions for power radiated and find the radiation resistance of a hold wave dipole. (8)			b.	Explain the concept of vector potential and retarded vector potential.						(7)

18.	Derive and draw the field pattern of arrays of two point sources with equal amplitude and in phase.

(OR)

19.	a.	Derive the expressions for far field components of a small loop antenna.					(10)

b.	For a 20 turn helical antenna operating at 3 GHz with circumference C = 10cm and the

spacing between the turns is 0.3, calculate the directivity and HPBW.					(5)

20.	Explain the construction, operation and design consideration for a rhombic antenna.

(OR)

21.	Explain the geometry of a log periodic antenna. Give the design equations of it.

22.	Explain the radiation from a Huygen’s source.

(OR)

23.	a.	Derive the relationship between dipole and slot impedances.							(10)

b.	Find out beam width between first nulls (BWFN) and power gain of a 2m paraboloid reflector operating at 6000 MHz.															(5)

24.	Derive the expression for the refractive index of the Ionosphere.

(OR)

25.	a.	Explain ground wave propagation.	b.	Write notes on diversity reception.