Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	ELECTRON DEVICES								              Time : 3 hours

Subject Code: 	EC283			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	The number of electrons per coulomb is _________.

2.	The radius of an electron is ______________.

3.	How semi conductors are classified?

4.	The Fermi level in an N – type material is _________.

5.	The diode current equation relating the voltage V and current I is given by _________.

6.	The input resistance of CB configuration is

a.	low		b.	moderate		c.	high

7.	Say True or False:	 = gm x rd

8.	List the various terminals of SCR

9.	Say True or False: LED consumes more power than the LCD.

10.	Say True or False: Life time of LCD is less than the LED.

PART – B  (5 x 3 = 15 MARKS)

11.	Find the speed and the kinetic energy of an electron after it has moved through a potential difference of 5000V.

12.	What is meant by Donor impurities?

13.	Define the term large signal current gain of a common base transistor.

14.	When the reverse gate voltage of JFET changes from 4.0 to 3.9V, the drain current changes from 1.3 to 1.6 mA. Find the value of trans conductance.

15.	State the applications of Tunnel diode.

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss the motion of an electron between two parallel plates under the influence of applied potential.

(OR)

17.	a.	Compare electrostatic deflection and electro magnetic deflection.					(6)

b.	The electron beam in a CRT is displaced vertically by a magnetic filed of flux density

2 x 10-4 wb/m2. The length of the magnetic field along the tube axis is the same as that of the electrostatic deflection plates. The final anode voltage is 800V. Calculate the voltage which should be applied to the Y-deflection plates 1 cm apart to return the spot back to the centre of the screen.																	(9)

18.	Explain the operation of PN junction diode when it is forward biased and reverse biased. Discuss its VI characteristics by the influence of temperature.

(OR)

19.	Explain Hall effect. How can the properties of semi conductor and determined with the aid of Hall Effect.

[P.T.O]

20.	a.	Explain the operation of NPN and PNP transistors.								(8)

b.	Explain the input and output characteristics of a transistor in CB configuration.		(7)

(OR)

21.	Explain active, saturation and cut off regions for a CB and CE transistor configuration.

22.	a.	Explain using neat sketches, the operation of UJT.								(7)

b.	What are the three main regions in a UJT static characteristics?					(4)

c.	Define interbase resistance and intrinsic stand off ratio as applied to a UJT.			(4)

(OR)

23.	a.	Explain the construction, operation and characteristics of JFET.					(9)

b.	Compare N-channel JFET and P – channel JFET.									(6)

24.	a.	Describe, with relevant diagrams, the construction of a LED and explain its working.	(10)

b.	Compare LED and LCD.														(5)

(OR)

25.	a.	Bring out the important difference between Zener breakdown and avalanche breakdown of a

PN Junction.																	(7)

b.	With the help of VI characteristics, show how a zener diode is used as a voltage regulator. (8)