Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	DIGITAL ELECTRONICS							              Time : 3 hours

Subject Code: 	EC284			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	(0.10110)2 = (____?________)10.

2.	A + A⋅B=___________.

3.	XOR gate is also called as an _________detector.

4.	A 2 input logic circuit with inputs I0 and I1 has a control input S. If the logic expression realized by the circuit is y=I0S+I1S, then the circuit is ____________.

5.	Number of flip – flops required to realize a MOD 68 counter is __________.

6.	The asynchronous inputs of a J – k flip – flop are __________and _________inputs.

7.	In a sequential circuit the present output depends on both the _________and __________.

8.	Draw the state diagram for the 2 bit counter.

9.	Emitter coupled logic belongs to __________family.

10.	Draw the circuit of CMOS INVERTER.

PART – B  (5 x 3 = 15 MARKS)

11.	Prove (A + B) (B + C) (C + A) = (A+B)(B+C)(C+A).

12.	Implement the logic expression f(a,b,c)=(abc+ a  {b c +ab c +a b  {c+ a b c )using 4 : 1 multiplexer.

13.	List the advantages of synchronous counter over that of asynchronous counter.

14.	Construct the state diagram for the system described in the state Table – 1 given below. Note that x is the input and A and B are the state variables.

| Present values x(t) A(t) B(t) | Next state A (t+1) B (t+1) |
| --- | --- |
| 0 0 1 0 0 0 0 1 1 0 1 0 1 0 1 1 0 0 1 1 0 1 1 1 | 0 0 1 1 1 0 0 1 1 1 1 0 0 1 0 0 |

15.	Draw the diagram of 2 input CMOS NOR gate.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Simplify the function using K – map f (a, b, c, d) =  m ( 1, 5, 6, 7, 11, 12, 13, 15)

Also identify the redundant term if any.											(10)

[P.T.O]

b.	A, 4 bit data word is used to generate a 7 bit composite Hamming code and is transmitted.

The bits received at the receiver are 0 1 1 1 0 0 0. Identify the error bit and correct the error if any.																		(5)

(OR)

17.	a.	Express the base – 10 number a = 12.1510 in IEEE 754 floating – point standard using single

precision.																	(8)

b.	Represent (482)10 decimal using (i) BCD code and (iii) excess three code.			(4)

c.	State De-Morgan’s theorems.													(3)

18.	a.	Design a binary to gray code converter for 3 bits and implement the same using gates. (8)

b.	Draw the Truth Table for full adder and obtain the expressions for SUM and carry and draw

the gate realization for it.														(7)

(OR)

19.	a.	Implement the  following function with NAND gates only

f (x,y, z) =  m (1, 2, 3, 4, 5, 7). 												(8)

b.	Realize a 8 : 1 multiplexer using two 4 : 1 multiplexers.							(7)

20.	Design a synchronous counter to count in the random sequence 0, 1, 2, 4, 5, 6, 0, 1 …….use J – k

flip – flops.

(OR)

21.	Draw the schematic of universal shift register constructed using D-flip-flops and multiplexers and explain its operation.

22.	Design a synchronous sequential circuit that detects three or more consecutive 1’s in a string of bits coming through an input line.

(OR)

23.	a.	Differentiate between Mealy and Moore model of sequential circuit.				(5)

b.	Explain state reduction through partitioning with an example.						(10)

24.	Draw the TTL NAND gate circuit with Totem pole output and explain its operation.

(OR)

25.	a.	List and explain CMOS characteristics.											(5)

b.	With the help of circuit schematic explain the operation of CMOS NOR gate.		(10)