Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	ADVANCED DIGITAL SIGNAL PROCESSING		              Time : 3 hours

Subject Code: 	EC301			         						                Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Draw the block diagram of a typical real-time DSP system and explain each

block briefly.																(12)

b.	Compute linear convolution : x(n)={1,2,3,4} and h(n)={1,0,1}						(8)

2.	a.	Find x(n) using residue method if

X(z)=zz−0.75z+0.5  												(8)

b.	Explain about Walsh-Hadamard transform.										(8)

c.	Calculate Walsh-Hadamard transform of (1, 2, 0, 3)								(4)

(OR)

3.	a.	Calculate the FFT of the given sequence using Decimation-in-time algorithm.

x(n)= {0,0,1,1,1,1,0,0}														(16)

b.	State and prove the following Properties of Z transform.

(i) Linearity (ii) Differentiation												(4)

4.	a.	A linear phase 9-point FIR filter(N=9) is characterized by the following frequency samples

H(k) = 1     k=0, 1, 2

= 0     k=3, 4

Calculate the filter coefficients using frequency sampling method.					(12)

b.	Write the procedure for designing a low pass filter using optimal method.				(8)

(OR)

5.	a.	Design a digital IIR low pass filter for the transfer function

H(s)=1s2+2s+1

using (1) Impulse Invariance method (2) Bilinear transformation method. Assume 3 dB cutoff frequency of 150Hz and sampling frequency  of 1.28 kHz.						(18)

b.	What are the design steps for digital IIR filter?									(2)

6.	a.	Describe interpolation and decimation by integer factors.							(10)

b.	Draw the block diagram of a 3 stage decimator used to reduce the sampling rate from

3072 kHz to 48 kHz. Assume decimation factors of 8, 4 and 2. Indicate the sampling rate at each stage. Also find the band edge frequencies at each stage if sampling frequency is 3072 kHz, Decimation factor is 64  and the frequency band of interest is 0-20 kHz.   			(10)

(OR)

7.	a.	Discuss in detail about the basic Least Mean Square (LMS) adaptive algorithm and its

practical limitations.														(14)

b.	How adaptive filter is used in noise cancellation?								(6)

8. 	Write notes about fixed point digital signal processors of TMS320 family with neat diagrams.

(OR)

9.	Explain about the time domain operations on musical sound processing.