Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	DIGITAL SYSTEM DESIGN						              Time : 3 hours

Subject Code: 	EC331			         						               Maximum Marks: 100

#### Answer ALL questions (5 x 20 = 100 Marks)

1.	Implement the following functions

F0= B C’ + B’ C + D’

F1=A’ B C + A’ C’ D + BCD + B’ C’ D’ + A B’ C D’

F2= ABD’ + AB’ D + B C’D’ + B’C’D + A’BCD + A’B’C D’

a.	Draw a PLA map to represent the PLA implementation of the functions, you have an unlimited number of product terms available.									(5)

b.	Draw a PAL map to represent the PAL implementation of the functions, you have an unlimited number of product terms available but each OR gate is restricted to the sum of 6 product terms.																(5)

C.	Draw the PROM structure for the above function									(10)

(OR)

2.	With neat sketch explain the different blocks in xilinx 4000 series.

3.	Design an asynchronous sequential circuit that has two inputs X2 and X1 and one output Z. When X1=0 the output Z is 0. The first change in X2 that occurs while X2 is 1 will cause output Z to the 1. the output Z will remain 1 until X1 returns to 0.

(OR)

4.	Design a synchronous counter for

5

22

0

4

6

75

3

1

Avoid lockout condition.  Use JK FF.

5.	a.	For the function

f = Πm(1,4,6,7,9,12,15)

i.	Find the minimum cost POS.												(2)

ii.	Draw the equivalent logic diagram											(3)

iii.	Write the VHDL coding using Data flow modeling. 							(4)

b.	Implement the octal to binary encoder using three 4 input OR gates. Also write the VHDL code for the above-implemented diagram using structural modeling.  	             		(2+2)

[P.T.O]

c.	i.	Write the Program to Count the Number of 1’s in a given 8-bit Number.		(3)

ii.	Write the Program for 4-bit Up Down Counter Using behavioral modeling.  		(4)

(OR)

6.	a.	Write the VHDL program for 4*1 multiplexer using function						(7)

b.	Write the VHDL program for n-bit ALU using procedures							(7)

c.	With an example explain the use of operator overloading							(6)

7.	Compulsory:

a.	Explain the different properties of Symmetric function							(10)

b.	Find the minimal contact network which realize the symmetric function

i.	S1,4(w,x,y,z)																(5)

ii.	S1,3(x1,x2,x3,x4,x5)														(5)

8.	a.	Write the Verilog program for

i.	Priority encoder															(6)

ii.	3*8 decoder																(6)

b.	Explain the different data operator types with an example							(8)

(OR)

9.	a.	Write the verilog coding for CMOS NAND gate using switch modeling				(7)

b.	How will you give gate delays in verilog? Explain with an example					(6)

c.	Write the Verilog program for 4-bit parallel in serial out shift register				(7)