Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	CMOS VLSI DESIGN								              Time : 3 hours

Subject Code: 	EC332			         						               Maximum Marks: 100

#### Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Design an 8 bit barrel shifter (i.e., arbitrary left to right rotate) using multiplexers. Explain

the limitations of your design.

b.	Design a high speed carry look ahead adder.

2.	a.	Explain:		a.	Body effect		b.	Channel length modulation

c.	Transmission gate characteristics

b.	Describe about Digital design domain and levels of abstraction

(OR)

3.	a.	Derive the expression for the current in the 5 region of operation for a CMOS device.

b.	Describe about static load invertors and Bipolar devices.

4.	a.	Explain the switching characteristics of CMOS?								(12)

b.	Discuss about charge sharing.												(8)

(OR)

5.	a.	With examples show the resistance and capacitances are estimated for VLSI circuits.	(8)

b.	Discuss about Design margining and CMOS Gate transistor sizing.				(12)

6.	a.	Describe about C2MOS Dynamic MOS and BicMOS logic.					(12)

b.	Bring about the characteristics of I/O pads.									(8)

(OR)

7.	a.	Explain about two phase clock structures and CVSL Logic structure.			(8)

b.	What are the constraints to be met by the design of CMOS logic gates? Explain them. Also

explain the any two types of layout used for CMOS logic design.				(6+6)

8.	a.	Discuss about CMOS testing strategies.										(10)

b.	Explain the chip design verification tools.									(10)

(OR)

9.	a.	Explain about system level test techniques.									(10)

b.	Discuss about manufacturing test principles.									(10)