Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	CELLULAR MOBILE COMMUNICATION			              Time : 3 hours

Subject Code: 	EC346			         						               Maximum Marks: 100

#### Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	What are the three basic propagation mechanisms?			       				(3)

b.	Explain the 2-ray ground reflection model and obtain an expression for the received power at a distance d from the transmitter.				        						(17)

2.	a. 	Define Grade of Service. How do you relate it with system capacity?	          	(4)

b.	Describe the frequency reuse techniques in allocating channel groups of a base station

within a system.							         							(12)

c.	Show that the frequency reuse factor for a cellular system is given by k/S; where k-average number of channels per cell, S- total number of channels available to cellular service provider.								         								(4)

(OR)

3.	a.	Assume each user of a single base station radio system average 3 calls/hour, each call lasting

an average of 5 minutes

i.	What is the traffic intensity of each user?  								(3)

ii.	Find the number of users that could use the system with 1% blocking if only one channel is available ( capacity = 0.153 Erlangs)	.			          		(3)

iii.	Find the number of users that could use the system with 1% blocking if five trunked

channels are available ( capacity = 1.36 Erlangs).		          				(3)

b.	Derive the Erlang B formula and determine the probability that a call is blocked.	(11)

4.	a.	Differentiate control and voice channels.		          						(4)

b.	Illustrate with a timing diagram how a call is made by a landline subscriber to a

mobile user.								         							(12)

c.	What do you mean by roaming?						          				(4)

(OR)

5.	a.	Why 900 MHz frequency is preferred for cellular mobile communication.		(6)

b.	Compare the characteristic feature of 1G,2G, 3G and 4G system.	        		(14)

6.	a.	Compare a zero crossing detector and a frequency discriminator.	        		(4)

b.	How is constant envelope and high power efficiency achieved in a cellular system  (12)

c.	What is Matched Filter?							         					(4)

(OR)

7.	a.	What are the factors that influence the choice of digital modulation?	         	(4)

b.	Explain the different types of channel coding techniques.    		         		(8)

c.	Enumerate the fundamentals of equalization and reduction in ISI in communication channels. (8)

8.	a.	Define Asynchronous Transfer Mode.										(3)

b.	Compare and contrast the features of FDMA, TDMA and CDMA.				(9)

c.	Describe cellular packet switched architecture.								(8)

(OR)

9.	a.	Explain various wireless data services.										(10)

b.	Describe the circuit switching and packet switching in wireless network.		(10)