Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	MODELLING OF DIGITAL SYSTEM DESIGN USING HDL	       Time : 3 hours

Subject Code: 	EC361			         						                Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	What are Symmetric Functions? Give one example.								(12)

b.	Draw a symmetrical network for three variables and explain how it is drawn.			(8)

2.	a.	Design a binary to GRAY code converter.										(10)

b.	Explain the structure of a PAL device (4 input and 3 input device)					(10)

(OR)

3.	a.	With a neat sketch explain in detail about the Input / Output block of an Xilinsc XC 4000

FPGA family.																(10)

b.	With a neat Block diagram describe in detail the Architecture used in an ALTERA FLEX 10

K family CPLD.															(10)

4.	a.	Design a MOD 6 counter using state table assignment and reduction method.			(12)

b.	Write short notes on ASM charts.												(8)

(OR)

5.	Explain:																		(6+6+4+4)

a.	Moore Machine.			b.	State table minimization

c.	Essential hazard

d.	Asynchronous sequential circuits working in fundamental mode

6.	a.	Describe the general roles of the following VHDL constructs in modeling switching algebra

operations.																(5+5)

i. Entity declaration								ii.	Architecture body

b.	Write the VHDL code for implementing a Nand gate.								(10)

(OR)

7.	a.	What is the importance of port map command? Using port map command write the VHDL

code for the full adder using half adder circuit.									(10)

b.	Write down the importance of (1). If statement (2) case statement in VHDL.			(10)

8.	a.	Write notes on the data types used in verilog HDL.								(10)

b.	Give the difference between logical operator and bit wise operator.					(10)

(OR)

9.	a.	Explain in detail about Switch Modeling elements used in verilog HDL.				(12)

b.	A simple module called d implements the following logic equations					(8)

out = (a . b) + c

The gate level implementation is shown in module D. The module contains two gate delays

5 and 4 time units. Write the verilog definition for module D with Delay.

a

e

#5

b

OUT

# 4

c

Module D