Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	ELECTRICAL TECHNOLOGY						               Time : 3 hours

Subject Code: 	EE205			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Write the expression for the speed of a d.c motor

2. 	Draw speed torque characteristics of d.c series motor

3. 	What is the condition for maximum efficiency of a transformer?

4. 	What is staggering in the construction of transformer?

5. 	Name the types of Alternator based on their rotor construction.

6. 	Under what condition terminal voltage of an alternator will be greater than induced emf?

7. 	An induction motor is generally analogous to ---------

8. 	Why is not possible for the rotor speed of an Induction motor to be equal to the speed of its rotating magnetic field?

9. 	How is the direction of a capacitor start Induction motor be reversed?

10.	What type of single phase induction motor would you use for Wet Grinder?

PART – B  (5 x 3 = 15 MARKS)

11. 	What are the necessary conditions for the generator to be self excited.

12. 	Define all day efficiency of a transformer

13. 	What are the requirements to be met with for paralleling of alternator?

14. 	What are the advantages of slip-ring Induction Motor over cage Induction Motor?

15. 	What is step angle?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain the various methods of excitation of dc machines.							(5)

b.	Why a starter is necessary? Explain the function of no volt coil and over load relays.	(10)

(OR)

17.	a.	Explain in details the construction and operating principle of a d.c motor.			(10)                                b.	What is back emf? Explain the significance of a back emf.                                                (5)

18.	a.	Write short notes on parallel operation of transformer.    			                    		(5)

b.	A 250 KVA, single phase transformer has iron loss of 1.8 KW. The full load copper loss is

2000watts.   Calculate				              				     						(10)

i). Efficiency at full load, 0.8 lagging p.f

ii) KVA supplied at maximum efficiency

iii) Maximum efficiency at 0.8 lagging p.f

(OR)

19.	a.	Explain the working of autotransformer and prove that when transformation ratio

approaches unity,  the amount copper used approaches smaller value. 				(8)

[P.T.O]

b.	The primary and secondary windings of a 500 KVA transformer have resistance of 0.4 ohm

and  0.001 ohm respectively. The primary and secondary voltages are 6600 V and 400 V respectively. The iron loss is 3KW. Calculate the efficiency on full load, the load power factor being 0.8 lagging.														(7)

20.	Explain the construction and principle of operation of 3-phase alternator.

(OR)

21.	a.	Explain why synchronous motor is not self starting                                  		         (5)

b.	 A 3 phase, 12 pole star connected alternator has 72 slots and 6 conductors per slot. If it is driven  at 500rpm, calculate the induced EMF (line to line). Assume coil span as 150 degree electrical and flux per pole 40 mwb.								        		(10)

22.	a.	Explain the torque Vs slip characteristics of a slip ring three phase induction motor.   	(10)

b.	Write a note on speed control scheme of 3 phase induction motor.                              	(5)

(OR)

23.	a.	Explain how the rotor rotates in three phase induction motor                                        (7)

b.	Explain rotor resistance starter of three phase induction motor                                      (8)

24.	a.	Explain principle of operation of DC and AC servomotor                           		       	(8)          b.	Write a brief note on shaded pole motor   				         		         		(7)

(OR)

25.	a.	What is stepper motor? Explain principle of operation and its working.				(8)                                    b.	Write a note on linear induction motor.											(7)