Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	DC MACHINES AND TRANSFORMERS			               Time : 3 hours

Subject Code: 	EE207			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The essential condition for stable parallel operation of two dc generators having similar characteristics is that they should have _______________.

2.	What is the best way of minimizing eddy currents in an armature?

3.	The armature torque of a dc motor is a function of its __________.

4.	What is the main disadvantage of Hokinson’s test?

5.	The main purpose of using core in a transformer is to __________.

6.	A transformer has negative voltage regulation when its load power factor is _________.

7.	The full load copper loss of a transformer is 1600W. At half – load the copper loss will be ____________.

8.	Why the transformer rating is in kVA?

9.	As compare to  -  bank, the capacity of the V – V bank of transformer is ______percent.

10.	The secondary of current transformer should never be left open under any circumstances. Why?

PART – B  (5 x 3 = 15 MARKS)

11.	What are the conditions necessary for the build-up of a self – excited shunt generator?

12.	A 4 – pole, 230 V series motor runs at 1000 rpm when the load current is 12A. The series field resistance is 0.8  and the armature resistance is 1.

The series field coils are now regrouped from all in series to two in series with two parallel paths. The line current is now 20A. If the corresponding weakening of field is 15%. Calculate the speed of the motor. Refer Fig. 1 and 2.

0.8

12A

1.0

Fig.1

230 V

1.0

0.4

20A

Fig.2

1.0

0.4

230 V

[P.T.O]

13.	Draw the phasor diagram of transformer for lagging power factor load.

14.	Mention the application of auto transformer.

15.	Draw the wiring diagrams for potential and current instrument transformer.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain the characteristics of DC shunt generator.

b.	A long shunt d.c compound generator drives 20 lamps, all are connected in parallel.

Terminal voltage is 550V with each lamp resistance as 500. Of Rsh = 25, Ra = 0.06 and Rse = 0.04, calculate the armature current and the generated e.m.f.

(OR)

17.	a.	Two shunt generators each with an armature resistance of 0.01 and field resistance of 20

run in parallel and supply a total load of 4000A. The e.m.f.s are respectively 210V and 220V. Calculate the bus-bar voltage and output of each machine.

b.	What are the methods of improving commutation? Explain.

18.	a.	A d.c shunt machine generator 250V on open circuit at 1000 r.p.m. Effective armature

resistance is 0.5, field resistance is 250, input to machine running as a motor on no-lad is 4A at 250V. Calculate speed of machine as a motor taking 40A at 250V. Armature reaction weakens field by 4%.

b.	A d.c shunt motor drives a centrifugal pump whose torque varies as the square of the speed.

The motor is fed from a 200V supply and takes 50A when running at 1000 r.p.m. What resistance must be inserted in the armature circuit in order to reduce the speed to 800 r.p.m? The armature and field resistances of the motor are 0.1 and 100 respectively.

(OR)

19.	a.	Explain the Hopkinson test on a pair of shunt motors.

b.	When running of no-load, a 400V shunt motor takes 5A. Armature resistance is 0.5 and

field resistance 200. Find the output of the motor and efficiency when running on full-load and taking a current of 50A. Also find the percentage change in speed from no-load to full-load.

20.	a.	Explain the operation of transformer on load.

b.	A 25 kVA transformer has 500 turns on the primary and 50 turns on the secondary winding.

The primary is connected to 3000V, 50Hz supply. Find the full-load primary and secondary currents, the secondary e.m.f and the maximum flux in the core. Neglect leakage drops and no-load primary current.

(OR)

21.	a.	Obtain the equivalent circuit of a transformer.

b.	The corrected instrument readings obtained from open and short circuit tests on 10 kVA,

450 / 120 V, 50 Hz transformer are:

O.C test:

V1 = 120V	;	I1	= 4.2A	;	W1 = 80 W	:	W1	and I1 were read on the low-voltage side

S.C. test:

V1 = 9.65V	;	I1 = 22.2A	;	W1 = 120W	with low voltage winding short-circuited.

Compute

i.	the equivalent circuit constants

ii.	efficiency and voltage regulation for an 80% lagging p.f load

iii.	the efficiency at half full-load and 80% lagging p.f load

22.	a.	Derive an expression for saving of copper when an auto-transformer is used and mention its

applications.

b.	A 5 kVA, 110 / 110V, single phase, 50 Hz transformer has full-load efficiency of 95% and

an iron loss of 50W. The transformer is now connected as an auto-transformer to a 220V supply. Of it delivers a 5 kW load at unity power factor to a 110V circuit, calculate the efficiency of the operation and the current drawn by the high-voltage side.

(OR)

23.	a.	Explain the parallel operation of 1 transformers.

b.	A 5 kVA transformer has 35W core loss and 40W copper loss at full load. It operates at rated kVA and 0.8 power factor lagging for 6 hours, one-half rated kVA and 0.5 power factor lagging for 12 hours and no-load for 6 hours. What is its all-day efficiency?

24.	a.	List the four possible ways of connecting a bank of three transformers for three-phase

service. State the advantages and disadvantages of each of the connection.

b.	A load of 1400 kVA at 0.866 p.f lagging is supplied by two three phase transformers of

1000 kVA and 500 kVA capacity operating in parallel. The ratio of transformation is the same in both : 6600 / 400 delta – star. If the equivalent secondary impedances are (0.001 + j 0.003)  and (0.0028 + j 0.005)  per phase respectively, calculate the load and power factor of each transformer.

(OR)

25.	a.	Describe the constructional features and explain the principle of operation of instrument

transformers.

b.	A 500 kVA, 3 phase 50 Hz transformer has a voltage ratio of 33 / 11 kV and is delta / star

connected. The resistances per phase are : high voltage 35, low voltage 0.876 and the iron loss is 3050 W. Calculate the value of efficiency at full-load and one-half of full-load resepectively (a) at unity p.,f and (b) 0.8 pf.