Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	ELECTRICAL MACHINE DESIGN					              Time : 3 hours

Subject Code: 	EE211			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is output equation of a Machine? Write the output equation for ac and dc machine.

2.	What are the factors that decide the choice of specific magnetic loading?

3.	Distinguish between real and apparent flux densities in the slot.

4.	What are the differences between leakage flux and fringing flux?

5.	How will you select the back and front pitch for a lap winding?

6.	Define space factor of a coil.

7.	In transformers, why the ℓv winding is placed near the core?

8.	Write the advantages and disadvantages of stepped core in transformers.

9.	What is short circuit ratio of a synchronous machine?

10.	Write the expression for length of air gap in a salient pole synchronous machine.

PART – B  (5 x 3 = 15 MARKS)

11.	Two machines A and B of same type have their linear dimensions in the ratio x : 1. They have some speed, flux density and current density. Find out the relation between their outputs.

12.	Calculate the unbalanced magnetic pull acting in a 20 kW dc motor. No. of poles = 2, Air gap length is 2.5mm. Area under each pole is 20 x 10 -3 m, and air gap flux density is 0.8 wb/m2. The rotor is vertically displaced by 0.4mm. The poles are located at 45 with the horizontal axis.

13.	Discuss the various factors affecting the choice of specific electric loading in the design of dc machines.

14.	What is the effect of frequency in the hysterisis and eddy current loss in transformers?

15.	Discuss the advantages of having large air gap in synchronous machines.

PART – C  (5 x 15 = 75 MARKS)

16.	Find the suitable diameter and length for a 3.7 kw, 440 volt, 50 Hz, cage type induction motor to run at a full load speed of 1440 rpm. Bav = 0.45 T, ac = 440 amp. conductors / cm full load efficiency = 85% and full load power factor is 0.85.

(OR)

17.	Determine the main dimensions, no. of poles of a dc shunt generator with following data.

Voltage rating	-	500V

Power			-	1000 kw

Speed			-	300 rpm

Bav				-	1 Tesla

ac				-	400 amp – conductors / cm

18.	Derive an expression to calculate the leakage permeance of parallel sided slot of a rotating machine.

(OR)

[P.T.O]

19.	a.	Write down the steps involved to determine the mmf of a section of a magnetic circuit. Also

explain why the calculation of mmf for air gap can not be generalized. 				(8)

b.	Calculate the apparent flux density of a teeth of armature of a dc machine. Given slot pitch =

24mm, slot width = 12mm, length of armature core including 5 ducts of 10mm each = 0.38m, stacking factor = 0.92. True flux density is 2.2 wb/m2 and mmf is 70000 AT/m. Take length of iron.

Li = (core length – Total width of ducts) x staking factors.						(7)

20.	Draw the winding diagram for a 4 pole 13 slots simplex wave winding with 13 commutator

segments. The no. of coil side per slot is 2.

(OR)

21.	A 6 pole dc generator is rated for 350 kw with 600 rpm, 440V. The armature dia is 0.75m and no. of coils is 288. Design a suitable commutator for the generator.

22.	Determine the dimensions of core and window for a 5 kVA, 50 Hz, 1, core type transformer. A rectangular core is used with long side twice as long as short side. The window height is 3 times the width. Volt per turn = 1.8 V, space factor = 0.2

Current density = 1.8 A/mm2, Bm = 1 wb/m2

(OR)

23.	Discuss the design of the transformer tank with cooling tubes.

24.	Discuss in detail, the various design steps involved in the design of rotor of an induction motor.

(OR)

25.	Design a cage rotor for a 40 Hp, 3 phase, 400 V, 50 Hz, 6 pole, delta connected induction motor having a full load efficiency of 87 % and full load pf of 0.85. Take D = 33 cm and L = 17 cm. Stator slots = 54, conductors per slot = 14. Assume suitable data.