Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	POWER ELECTRONICS							              Time : 3 hours

Subject Code: 	EE213			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	BJT is a _______________Controlled device.

2.	Draw the circuit symbol of GTO

3.	The average value of dc voltage obtained in a six pulse converter is _______________

4.	Which type of transformer is used in firing circuit?

5.	First quadrant operation of a Step – Up converter allows power flow from_______________

6.	Give the classifications of commutation circuits in dc choppers.

7.	Series inverters are extensively used in _______________

8.	In single – pulse modulation of PWM inverters, third harmonic can be eliminated if pulse width is equal to _______________

9.	A device which converts input power at one frequency to output power at a different frequency with one – stage conversion is called a _______________

10.	Give the classifications of SMPS.

PART – B  (5 x 3 = 15 MARKS)

11.	What is meant by Second Breakdown of BJTs?

12.	What is the effect of free wheeling diode?

13.	What is forced commutation? List out its types.

14.	Give the advantages of Current Source Inverter.

15.	What is a Cyclo converter? Enumerate some of its applications in industries.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain in detail the Principle of operation of IGBT and also explain its characteristics with neat sketches.

(OR)

17.	With neat sketches, explain the working of a Power BJT and draw the VI Characteristics.

18.	Explain the effect of source impedance on the performance of 1 Φ full converter.

(OR)

19.	Explain the working principle of 3 phase full converter with neat sketches.

20.	Describe in detail the working of voltage commutated chopper in different modes.

(OR)

21.	Explain the various types of firing circuits used in DC – DC choppers.

22.	Explain in detail the principle of operation of a 1 Φ capacitor commutated Current Source Inverter with R load.

(OR)

23.	With neat sketches explain the working of PWM Inverter

24.	Explain in detail the working of 3 Φ – 3 Φ cyclo converter.

(OR)

25.	Explain any three types of SMPS.