Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	POWER SYSTEM ANALYSIS						              Time : 3 hours

Subject Code: 	EE216			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is per unit representation?

2.	Write the formula to get per unit impedance value?

3.	State the properties of  ‘tree’ of a graph?

4.	Define cut set in a connected graph?

5.	What are sequence network?

6.	Distinguish between symmetrical and unsymmetrical short circuits?

7.	What are the quantities specified at a generator bus?

8.	What is a Jacobian matrix?

9.	State one merit of equal area criterian?

10.	Write the swing equation of two machine system?

PART – B  (5 x 3 = 15 MARKS)

11.	Given the base MVA and base KV of a three phase power system derive the expression for base impedance?

12.	What are the applications of bus impedance matrix?

13.	How symmetrical components can be used to represent unbalanced vectors?

14.	What is the necessity for slack bus?

15.	What are the approximations in stability analysis?

PART – C  (5 x 15 = 75 MARKS)

16.	Two generators rated at 10MVA, 13.2KV and 15MVA, 13.2KV are connected in parallel to a bus bar. They feed supply to two motors of input 8MVA and 12MVA respectively. The operating voltage of motors is 12.5KV. Assuming base quantities as 50MVA and 13.8KV. Draw the reactants diagram. The percentage reactants for generators is 15% and that for motor is 20%

(OR)

17.	The one line diagram of an unloaded power system is shown in figure(1).  Reactances of  the two sections of transmission line are shown in the diagram. The generators and transformers are  rated as follows.

Genertor 1:20MVA,13.8KV, X”=0.2 p.u.

Genertor 2:30MVA,18KV, X”=0.2 p.u.

Genertor 3:30MVA,20KV ,X”=0.2 p.u.

Transformer 1: 25MVA, 20KV, X”=0.2 p.u

Transformer 2: Single phase units each rated 10MVA, 127/18KV, X=10%

Transformer 3: 35MVA, 220Ү/22Ү KV, X=10%

Draw the impedance diagram, choose a base of 50 MVA, 13.8KV in the circuit of generator 1.

[P.T.O]

Fig(1)

18.	Form the bus impedance matrix for the network shown in the figure(2)

Fig(2)

(OR)

19.	Explain the formation of Ү bus by singular transformation.

20.	Derive an expression for total power in a three phase system in terms of the sequence components of voltages and currents.

(OR)

21.	A three phase transmission line operating at 33KV and having a resistance and reactance of 5Ω and 20Ω respectively is connected to a generating station bus bar through a 15MVA step up transformer which has a reactance of 0.06p.u. Two generators one 10MVA having 0.1p.u reactance and another 5MVA, 0.075p.u reactance are connected to the bus bars. Calculate the short circuit MVA and fault current when a 3phase short circuit occurs at

- the high voltage terminals of transformers and  (b) at the load and of the transmission line.

22.	Draw the flow chart for load flow solution by Gauss-Seidal iterative method and explain

(OR)

23.	Derive the mathematical model of phase shifting transformer to be used in power flow analysis.

24.	a.	State the bad effects of instability. Distinguish between steady state and transient stabilities.

b.	Write short notes on assumptions made in deducing equal area criteria and validity of the assumption.

(OR)

25. 	Give the flow chart for transient stability analysis of a multi machine power system and explain each step.