Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	TRANSMISSION AND DISTRIBUTION				              Time : 3 hours

Subject Code: 	EE218			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In overhead system, the comparison of various systems is made on the basis of maximum voltage between _________.

2.	The economic transmission voltage is one for which the transmission cost is __________.

3.	The d.c resistance of the line conductor is ________than it’s a.c resistance.

4.	If the spacing between the conductors is increased, the inductance of the line _________.

5.	The length of the short transmission line is about __________km.

6.	In medium transmission lines, effects of _________are taken into account.

7.	The metallic sheath is provided over the insulation to protect the cable from ________.

8.	If the length of a cable is doubled, its capacitance is _________.

9.	What do you infer, if the string efficiency is 100%.

10.	Sag is provided in overhead lines so that __________.

PART – B  (5 x 3 = 15 MARKS)

11.	Write the operating transmission voltages of the network.

12.	Define skin effect.

13.	Write the significance of the reactive power.

14.	What are the types of overhead line insulators?

15.	Write about the types of tariff followed in India.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	What is the need of primary distribution in an electric supply scheme?				(7)

b.	Discuss the reasons for dc transmission over a.c transmission.						(8)

(OR)

17.	Discuss the recent trends of power transmission by EHVAC and HVDC systems.

18.	Deduce an expression for line to neutral capacitance for a 3-phase overhead transmission line when the conductors are (a) symmetrically placed	(b) un symmetrically placed but transposed.

(OR)

19.	The three conductors of a 3 wire are arranged at the corners of a right angled isosceles triangle. If each equal side of this triangle is 2m, find line – to – neutral capacitance per km. Take the diameter of each conductor as 1.24 cm. The conductors are transposed at equal intervals.

20.	A 3-phase, 50Hz transmission line 100 km long delivers 20 MW at 0.9 pf lagging and at 110 kV. The resistance and reactance of the line per phase per km are 0.2 ohm and 0.4 ohm respectively, while capacitance admittance is 2.5 x 10-6 siemen / km/ ph.

a.	Calculate the current voltage at the sending end.

b.	Efficiency of transmission. Use nominal T method.

[P.T.O]

21.	Write the significance of various compensation schemes and corona in the transmission system.

22.	a.	Derive for the dielectric stress in a single core cable.								(7)

b.	Describe about the grading of cables.											(8)

(OR)

23.	a.	Derive an expression for the thermal resistance of dielectric of a single – core cable.	(8)

b.	What do you infer from the permissible current loading of an underground cable?	(7)

24.	Discuss the various conductor materials used for over head lines. What are their relative advantages and disadvantages?

(OR)

25.	Discuss about various tariff structure of the electrical energy.