Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	PROTECTIOIN AND SWITCH GEAR				              Time : 3 hours

Subject Code: 	EE219			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	An over current relay having a current setting of 125% is connected to a supply circuit through a current transformer of ratio 400/5. The pick up value is ________

2.	Protection against unbalanced supply voltage is provided by ________relays.

3.	The number of pilot wires required for protecting 3-phase transmission lines using translay system of protection is ___________.

4.	To protect the power transformer with star-delta connection against fault, the current transformers secondary will have _________connection.

5.	The voltage that appears across the contacts after the circuit breaker is opened is called as _____.

6.	Switching of capacitance loads (capacitor loads or unloaded lines) can lead to arc restriking in circuit breakers). Say True or False.

7.	The power factor of the arc in circuit breaker is always ___________.

8.	The circuit breaker preferred when the current to be interrupted is low with high voltage arcs is _________.

9.	Two voltage surges are specified as 1/50 S and 3/50 S respectively. Which is harmful?

10.	In substations, the most commonly used type of arrester is ___________.

PART – B  (5 x 3 = 15 MARKS)

11.	What is over current relay? Draw the time-current characteristics of an inverse current relay.

12.	Over voltage protection is not provided in turbo-alternator sets. Why?

13.	In a short circuit test on a circuit breaker, it was obtained that the peak re-striking voltage is

100 V and the time to reach it as 50 S. Determine the average RRRV and the frequency of oscillations.

14.	Define: a.rated symmetrical interrupting current and (b) rated momentary current, associated with circuit breakers.

15.	What is voltage surge? Draw the typical wave form of 1/50 S lightning surge.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Discuss about the essential qualities of protective relays.							(5)

b.	Explain with the help of neat diagram the construction and working of Induction type

directional power relay.														(10)

(OR)

17.	Explain with suitable sketches, the microprocessor based distance relay for protection of transmission line.

[P.T.O]

18.	Describe with help of neat sketches the graded time lag protection of a radial feeder. What are the disadvantages of such a protection in case of

a.	Parallel feeder	and		b.	Interconnected lines

(OR)

19.	With neat sketches, describe the Merz-price circulating current system for the protection of Transformers.

20.	Explain briefly the arc phenomenon and various methods of arc extinction in a circuit breaker.

(OR)

21.	Explain the following phenomenon briefly: a.	Current chopping and 	b. Capacitive current

Breaking.

22.	Using simple electrical circuit, explain the principle of interruption of surges during short circuits and switching operation.

(OR)

23.	Discuss the principle of operation of an air-blast circuit breaker. What are the advantages and disadvantages of using air as the arc quenching medium?

24.	a.	Discuss the various causes of over voltages on a power system.					(7)

b.	How do earthing screen and ground wires provide protection against direct lightning

strokes?																		(8)

(OR)

25.	Giving the principle of operation of a lighting arrester, explain the construction and working of Thyrite arrester.