Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	INTRODUCTION TO ELECTRICAL MACHINES AND DRIVES									              											Time : 3 hours

Subject Code: 	EE221		         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the working principle of DC generator?

2.	What is the condition for maximum efficiency in a transformer?

3.	Synchronous motor can not be started directly. Why?

4.	What are the different ways to regulate the voltage of three phase alternator?

5.	How the induction motor is co-related with transformer?

6.	Express the %slip of induction motor.

7.	Name the windings in stepper motor.

8.	Write one of the basic requirements of a servomotor.

9.	What are the classifications of electric drives?

10.	What type of control schemes employed in modern electric drives?

PART – B  (5 x 3 = 15 MARKS)

11.	Derive the EMF equation of a transformer.

12.	What is meant by hunting?  How the hunting effect can be reduced?

13.	What is an induction motor?  How does the rotor rotate?

14.	List out the applications of linear induction motor.

15.	Write the basic features of control techniques for an electric drive.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the construction, principle of operation of a DC motor with a neat sketch.

(OR)

17.	a.	Describe the performance characteristics of a DC shunt generator.                     		(8)

b.	In a 25KVA, 2000/200V, single phase transformer, the iron and full load copper losses are 350W and 400W respectively.  Calculate the efficiency at unify power factor on (i) full load   (ii) half load.                                                										(7)

18.	Explain the operating principle of three phase alternator with a neat construction.

(OR)

19.	What is meant by synchronization?  Explain the various methods of synchronization with its advantages.

20.	Discuss the construction details of three phase induction motor.  Also differentiate the squirrel cage and slip ring rotors.

(OR)

[P.T.O]

21.	a.	Write short notes on single induction motor.                                                         		(8)

b.	The power input to the rotor of 400V, 50Hz, 6pole, 3 phase Induction motor is 75KW. The

rotor electromotive force is observed to make 100 complete alteration per minute. Calculate (i) Slip (ii) Rotor Speed (iii) rotor copper loss per phase

(iv) Mechanical power developed. 	                                                 					(7)

22.	Explain the construction and working of a variable reluctance stepper motor.

(OR)

23.	Write short note on;																(8+7)

a.	DC servomotors                              b.	AC servomotors

24.	With a neat diagram explain the microprocessor based control of drives.

(OR)

25.	a.	What are the factors taking into account in the selection of drives?               			(5)

b.	Explain the control scheme for steel rolling mills with schematic diagram. 			(10)