Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	ELECTRONIC CIRCUITS							               Time : 3 hours

Subject Code: 	EE225			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define voltage regulation of a power supply.

2.	What is the RMS valve of Half Wave Rectifier?

3.	Ideal common collector amplifier configuration will have----------- gain

4.	State any one application of class C Amplifier

5.	If input voltages v1 =100mV & v2= 50 mV is applied to a differential amplifier. Calculate Common mode signal.

6.	Draw the circuit of FET input stage DIFF. AMP circuit

7.	State barkausen criterion.

8.	Draw the (Electrical) equivalent of Crystal

9.	Which electronic equipment utilizes Sawtooth wave form as time base

10.	Write any one application of Astable & Monostable Multivibrator

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the circuit of bridge rectifier with R load. Also draw the relevant wave Forms.

12.	For the circuit of Fig 1. Calculate the Q point. Assume VBE =0.3, Beta=100.

13.	Calculate the CMRR in dB , if Ac=10.0  and Ad=10000

14.	An amplifier has gain of 1000 without feedback. Voltage series feedback is applied with Beta= 0.01. Find the overall gain

15.	Design a simple Integrator for square wave of 10.0 KHz signal

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the working of bridge rectifier circuit and derive the max. Conversion efficiency (with out filter).  Assume the diodes are ideal.

(OR)

17.	Explain the working principle of series voltage regulation and design the component valves for the specification given below:

Vo=10V

Io= 0-50mA

Vi=25-20V

[P.T.O]

18.	(a) 	Two amplifiers are identical having 100dB gain and 20 kHz bandwidth are cascaded.

Calculate the overall bandwidth and gain.

(b) 	State the advantages of Darlington pair connection through example.

(OR)

19 .	For the amplifier shown in Fig .2, find the voltage gain. Assume suitable Model for FET

20.	(a)	Describe the procedure for determining the CMRR of DIFF .Amp.  What is    the

significance of CMRR.

(b) Write the short notes on Tuned Amplifiers.

(OR)

21.	Draw the FET input DIFF AMP circuit and analyze through small signal model.

22.	Explain the operation of Wein bridge oscillators and derive the formula for oscillation

(OR)

23.	(a) 	What are the advantages of negative feedback.

(b) 	Prove that (through block diagram approach) Amplifier with voltage feedback will increase

Input Impedance and reduce the Output Impedance.

24.	Explain the working principle of Astable multivibrator and also design the component valves for the specifications

Vcc=10V, Frequency= 1HHz, beta= 50, Ic=4mA

(OR)

25.	Explain the working principle of UJT relaxation oscillator.