Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	ENERGY SYSTEMS								              Time : 3 hours

Subject Code: 	EE226			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Name few thermal plants available in Tamilnadu.

2.	What are the non – conventional energy sources available for power generation?

3.	Mention the applications of high frequency induction heating.

4.	What are the two laws of illumination?

5.	Define the term Electric drive.

6.	Define the term scheduled speed in traction

7.	What are the different forms of High voltage?

8.	What is the purpose of Deltatron?

9.	What are the tests normally conducted in high voltage apparatus?

10.	What is withstand voltage?

PART – B  (5 x 3 = 15 MARKS)

11.	Define the following terms

a.	Load Factor		b.	Diversity Factor	c.	Plant Use Factor

12.	Draw and explain the polar curves.

13.	Draw the circuit diagram of Bridge Transition.

14.	Explain the measurement of radio interference voltage.

15.	Draw and explain the circuit diagram of cascading transformers.

PART – C  (5 x 15 = 75 MARKS)

16.	Draw a typical layout of a thermal power plant and describe the functions of various components.

(OR)

17.	What is a nuclear reactor? Draw a layout of a nuclear power plant and describe the functions of various components.

18.	Explain, with neat sketches, describe the construction, principle of operation and application of Ajax Wyatt Furnaces.

(OR)

19.	Describe the construction, principle of operation and application of

a) Sodium Vapor lamp b) Mercury Vapor lamp c) Fluorescent lamp.

20.	Sketch the typical Speed - Time curves for a) Mail line service 2) Sub Urban service with electric traction.

(OR)

21.	Explain various methods of electric braking. State the conditions to be fulfilled for each method of braking.

[P.T.O]

22.	Explain how a sphere gap can be used to measure the peak value of voltages. What are the parameters and factors that influence such voltage measurement?

(OR)

23.	Describe, with a neat sketch, the working of a Van de Craaff generator.

24.	What are the different power frequency tests done on insulators and bushings? Mention the procedure for testing.

(OR)

25.	Explain the method of impulse testing of high voltage transformers. What is the procedure adopted for location the failure?