Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	ELECTRIC CIRCUITS								              Time : 3 hours

Subject Code: 	EE245			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State ohm’s law

2.	Resistors R1 and R2 are connected in parallel. What will be the equivalent resistance?

3.	A voltage of v(t) =340 sin 314t is applied to a resistor and current is i(t) =42.5 sin 314t. Find the value of ohmic resistance?

4.	Draw the power triangle for ac circuits.

5.	Equation to find the number of mesh currents in a circuit is__________________

6.	Define mesh current.

7.	Superposition theorem cannot be applicable for non-linear circuits. Say True or False

8.	The maximum power transferred to the load resistor occurs when load resistance is equal to _______________

9.	Define bandwidth

10.	Impedance of series RLC circuit at resonance is minimum.  Say True or False

PART – B  (5 x 3 = 15 MARKS)

11.	Find the equivalent resistance between A and B for circuit given below.

12.	Draw the wave form for the voltage and current equations given below.

v(t)=10 sin t

i(t)=5 sin (t-90)

13.	State Thevenin’s Theorem

14.	A series RLC circuit with R=10  and L=10 mH and C=1 µF has an applied voltage of 200V at resonant frequency. Calculate the resonant frequency and Q-factor.

[P.T.O]

15.	Convert the following voltage source in to Current source.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Determine ix in the network shown in figure using Kirchhoff’s law.					(7)

+

-

8ix

b.	Find the equivalent resistance between X and Y for network given in figure.			(8)

(OR)

17.	a.	State and explain Kirchhoff’s Current law.										(3)

b.	Determine the power delivered by the batteries of figure given below.				(12)

18.	Find the average and rms value of the trapezoidal wave shown below.

V

20

t

T

(OR)

19.	a.	A series RLC circuit has R=25 , L=0.221 H and C=66.3 µF and is supplied at 150V, 60

Hz. Find i) the equivalent Impedance 											(8)

ii) the power factor

iii) the current

iv) the real power and reactive power

b.	In the circuit given in figure, the voltage across the 5  resistor is 60V. Find the total current supplied by the source.														(7)

20.	a.	In the network shown in figure, the voltage V1=, find V2 such that the current in the

(1+j1) branch is zero.															(10)

b.	Find the mesh currents for the network given in figure using mesh analysis.			(5)

(OR)

21.	In the network of figure given below, find the node voltages V1 and V2 using nodal analysis.

22. 	Find the current in the 1 ohms resistor using Northon’s Theorem for the circuit given below.

(OR)

23.	Find the maximum power transferred to the load ZL of figure given below using maximum power transfer theorem.

24.	Derive an expression for resonant frequency of a series RLC circuit. Also find the expression for lower and upper cutoff frequencies.

(OR)

25.	a.	Determine the value of the capacitance C in order that the circuit of given figure is resonant

at 6366 Hz.																		(8)

b.	A variable inductive reactance is connected in parallel with series combination of R and capacitive reactance XC. For resonance derive an expression for XL in terms of R and XC. (7)