Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	ELECTRO MAGNETIC FIELDS					              Time : 3 hours

Subject Code: 	EE248			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write the expression for differential volume in spherical coordinate system.

2.	If the curl of a vector field is zero then the field is ______.

3.	D =?E

4.	Write Poisson’s equation.

5.	Define magnetic flux density.

6.	Write the expression for inductance of a toroid.

7.    State Faraday’s law.

8.	Differentiate between generator e.m.f and transformer e.m.f .

9.	Define electromagnetic wave.

10.	Write the expression for reflection coefficient at the sending end of a transmission line.

PART – B  (5 x 3 = 15 MARKS)

11.	State Stoke’s theorem.

12.	Define electric potential and derive the expression for potential difference between two points.

13.	Write the expressions for magnetic boundary conditions.

14.	Derive the expression for displacement current density.

15.	A copper sheet has σ = 5.8 × 107 siemens/m and μr = 1. Calculate the skin depth at the frequency

of 100 Hz.

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	Derive the relationship between Cartesian and cylindrical coordinate variables.           (5)

b.	Define divergence of a vector. Mention its properties. Write the expression for divergence of a vector field in all the three coordinate systems.                                                  		(10)

(OR)

17. 	a.	Find the angle between the vectors, A =2ax+4ay−az and B =3ax+6ay−4az using

dot product and cross product .                                                                          		(5)

b.	Find the curl of the following vector fields.								          (5+5)

i)	A =exyax+sin(xy)ay+cos2(xz)az ii)  	B =ρz2cosφ aρ+zsin2φ az

18. 	a.	Four point charges each 20 μC are placed at X and Y axes at ±4m. Find the force on the

charge of 30 μC at (0,0,3 m).                                                                                      	(7)

b.	An infinitely long line charge of uniform density λl C/m is placed along Z –axis. Find the

expression for electric field intensity at a point in Y –axis, which is ‘a’ meters away from Z –axis.  																		(8)

(OR)

[P.T.O]

19. a.	Given the potential V=10sinθcosφr2 (volts), find the electric field intensity at (2,π2,0).	(8)

b.	Find the expression for capacitance of a spherical capacitor.                                           	(7)

20. 	a.	A radial magnetic field H =2.39×106cosφρaρ (A/m) exists in free space. Find the

magnetic flux crossing the surface defined by −π4 ≤φ≤π4, 0 ≤z≤1m.                     		(7)

b.	Find the expressions for magnetic field intensity in the four regions of a co-axial cable using

Ampere’s circuital law.                                                                                               		(8)

(OR)

21. 	a.	A circular loop located at  x2+y2=25,z=0 carries a current of 8 A along aφ. Find the

magnetic flux density at (0,0 ,6 m).                                                                             		(5)

b.	Derive an expression for the torque experienced by a rectangular current loop placed in a

magnetic field.                                                                                                      		     (10)

22. 	a.	Obtain an expression for the e.m.f generated in Faraday’s disc generator.                  		(8)

b.	A circular loop having radius 0.1 m and resistance 5 Ω lies in the z=0 plane. The circular

loop is subjected to a magnetic field of B =0.2sin(103t)az (Wb/m2). Find the current in the loop due to induced e.m.f.                                                                                             		(7)

(OR)

23. 	Derive the Maxwell’s equations both in integral and differential forms.

24. 	a.	Derive the generalized  wave equation.                                                                                	(8)

b.	Derive the expressions for phase shift and intrinsic impedance offered by dielectric medium

to electromagnetic waves.                                                                                           		(7)

(OR)

25. 	a.	Derive the Poynting’s theorem. Give the interpretation for each term.                         	    (10)

b.	An uniform electromagnetic wave has rms magetic field value as 0.01 A/m. Calculate the

power density in free space.                                                                                                   (5)