Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	ELECTRICAL ENGINEERING						               Time : 3 hours

Subject Code: 	EE296			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the elements of electric circuits?

2.	State ohm’s Law.

3.	Write the expression for series resonant frequency.

4.	Write the expression for relationship between mutual inductance (M) and self inductance of two coils (L1, L2).

5.	What are the types of DC motors?

6.	What is the expression for step angle?

7.	Define Luminous flux.

8.	What is the application of incandescent lamp?

9.	What are the types of moving iron instruments?

10.	What are the accessories used in domestic wiring?

PART – B  (5 x 3 = 15 MARKS)

11.	A resistance of 100 Ω is connected in series with a 50 μF capacitor to a supply at 200V, 50 Hz.  Find the power factor.

12.	State Thevenin’s theorem.

13.	List out the applications of transformers.

14.	Define Luminaries.

15.	What are the types of wiring?

PART – C  (5 x 15 = 75 MARKS)

16.	An electrical network is arranged as shown in fig.  Find

(a) 	The current in branch AF     (b) The power absorbed in branch BE

(c) 	P.D. across branch CD.

A

F

9 OHMS

18 OHMS

B

D

5 OHMS

22 OHMS

11 OHMS

14 OHMS

13 OHMS

E

1 OHMS

C

24 V

(OR)

[P.T.O]

17.	Write the mesh equations for the network shown in fig. by inspection and find the power absorbed by 8 Ω resistor.

18.	Compute the current through 23 Ω resistor of the circuit shown in fig. by using   super position

theorem.

(OR)

19.	A series RLC circuit with R = 10 Ω , L = 1 mH and C = 1000 PF is connected  across a sinusoidal source of 20 V with variable frequency a ) compute the  resonance frequency of the circuit b) Find the Q factor of the circuit at the  resonant frequency  c) Determine the half power frequencies.

20.	Explain the construction and working principle of 3 Φ Induction motors.

(OR)

21.	Explain the constructional details and working principle of reluctance motor. Enumerate its advantages and disadvantages.

22.	Explain about different types of lighting systems.

(OR)

23.	Draw and explain the working of fluorescent lamp circuit.

24.	What is the need of earthing?  Explain its types in detail.

(OR)

25.	Explain the construction and working principle of Dynamo meter type wattmeter.