Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	INDUSTRIAL ELECTRONICS AND INSTRUMENTATION  Time : 3 hours

Subject Code: 	EE311			         						                Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Explain the working principle of photo conductive cell.                                   		(8)

b.	Explain the basic structure of a PLC which is made to fit an industrial environment. Bring out the significance of ladder translation of programming. 						(12)

2.	a.	What is the principle of a resistive transducer and explain the instrumentation  for

displacement measurement using resistive transducer principle.          				(12)

b.	Explain the working principle of piezo electric transducer for force measurement.     	(8)

(OR)

3.	a.	Describe the operation of an LVDT for measuring displacement.  How is its operation

depends on the position of the core.                                            					(10)

b.	Write the principle of a capacitive transducer and describe the method of  measurement of

pressure using capacitive transducer.                              							(10)

4.	a.	Explain the working principle of an incremental shaft encoder for measurement of angular

displacement.                                                         									(8)

b.	Write the principle of a Moire Fringe transducer and explain the application of Moire Fringe

transducer to sense angular motions.                                							(12)

(OR)

5.	a.	Explain with a schematic diagram, the vibrating string type of strain gauge for measurement

of force.  Also state its natural frequency and resonant frequency.                               	(16)

b.	Write a short note on IC sensor for measurement of temperature.                 			(4)

6.	a.	Draw the block diagram for the microprocessor based firing circuit of a  thyristor.  	(8)

b.	Draw and explain the system hardware block diagram of AC voltage measurement.  Also

state the requirements of system software.                								(12)

(OR)

7.	a.	Explain with a suitable block diagram the feedback control system of an AC  motor to

monitor and control the speed at constant value.                         					(10)

b.	Draw the schematic diagram of microprocessor based over current protective system and

explain with flow chart.                                                           						(10)

8.	a.	Explain the salient features of smart transducers with basic block diagram.  Bring out their

advantages over conventional transducers.

(OR)

9.	a.	Explain the significance of self diagnosis of faults and calibration in

smart  transducers.                                                                                                  		(8)

b.	Explain the measurement setup to measure the flow using smart transducer.            	(12)