Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	POWER SEMICONDUCTOR DEVICES				              Time : 3 hours

Subject Code: 	EE328			         						                Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Discuss the switching characteristics of power diode.								(10)

b.	Discuss the structure and V-I characteristics of schottky diode.						(10)

2. 	a.	Explain the working principle and V-I characteristics of SCR.						(10)

b.	A thyristor string is formed with voltage and current ratings of 6 KV and 4 KA. The available  voltage  and  current  rating of thyristor are   1.2 KV  and   1 KA  respectively.

The string efficiency is 90% for both series and parallel operation.

Determine,

i.	number of thyristors to be connected in series and parallel.

ii.	calculate the values of R and C, if the maximum blocking current is 15 mA

and ∆Qmax  = 25 μC.  														(10)

(OR)

3. 	Discuss the operation of RC triggering and UJT triggering circuits of SCR.

4. 	Explain the turn-on and turn-off characteristics of GTO.

(OR)

5.  a.	Describe the V-I Characteristics of TRIAC.										(10)

b.	Explain how GTO is protected from over current.								(10)

6. 	Explain the working principle and V-I Characteristics of any one type of MOSFET.

(OR)

7. 	a.	Discuss the base drive mechanism of BJT.										(10)

b.	Write short note on on-state losses of BJT.										(10)

8. 	a.	Compare the performance characteristics of IGBT and BJT.						(10)

b.	Discuss any one of the gate drive circuit used in IGBT.							(10)

(OR)

9. 	Write short notes on the following:												(10+10)

a.	Power integrated circuit			b. SIT