Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	POWER ELECTRONICS - I							              Time : 3 hours

Subject Code: 	EE329			         						                Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Explain the effect of source inductance on output voltage with necessary equations in converter circuits.																	(10)

b.	Explain with necessary waveforms Extinction angle control schemes for improvement of

power factor.																(10)

2.	a.	Explain with a neat power diagram and associated waveforms the operation of a single

phase half-wave controlled bridge converter with RL load.							(10)

b.	Describe briefly the various performance parameters of a single phase controlled rectifiers

with necessary equations													(10)

(OR)

3.	a.	Explain with necessary waveforms the operation of a single phase fully controlled bridge

converter with R load.														(10)

b.	A single phase fully controlled bridge converter supplies an inductive load. If the supply voltage is 230V, 50Hz and the firing angle is maintained at 30º, calculate Fundamental power factor, supply power factor, harmonic factor and voltage ripple factor.							(10)

4.	a.	Explain with necessary waveforms the continuous current operation of a three phase fully

controlled thyristor bridge converter with RL load.								(10)

b.	A three phase fully controlled converter is operating at 415V, 50 Hz supply and deriving an inductive load with load resistance of 100Ω. Determine the Displacement power factor, Distortion factor, Harmonic factor and Supply power factor for a firing angle α = 60º. 			(10)

(OR)

5.	a.	Explain with necessary waveforms the operation of a three phase half controlled thyristor

bridge converter with RL load.												(10)

b.	A three phase half controlled converter is connected to a 380V (line) supply. The load

current is constant at 32A, and is independent of firing angle. Calculate the average load voltage at firing angle 45º, given that the thyristors have a forward voltage drop of 1.2V and what will be the average power dissipation in each thyristor.										(10)

6.	a.	Explain with a neat diagram and waveforms the operation of a single phase bridge inverter

with RL load.																(10)

b.	Describe the various harmonics reduction techniques used in inverters.				(10)

(OR)

7.	a.	Explain with a neat diagram and waveforms the operation of a three phase bridge inverter

with resistive load in 180º conduction mode.									(12)

b.	Describe the various Inverter circuit design aspects.								(8)

8.	a.	Describe with a neat diagram the Zero current Switching resonant converters.			(10)

b.	Describe with a neat diagram the operation of a two quadrant zero voltage switching

resonant converters.														(10)

(OR)

9.	a.	Explain with a neat diagram the operation of a resonant dc link inverter.				(12)

b.	Compare Zero Current Switching and Zero Voltage Switching resonant

converter topologies.														(8)