Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	POWER ELECTRONICS - II						              Time : 3 hours

Subject Code: 	EE330			         						                Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

Explain the operation of the step down chopper with associated output voltage and current waveform for RLE load. Also derive the formula for minimum and maximum values of current.

2.	a.	Draw and explain the circuit diagram of Cuk regulator and also derive the expression for

average output voltage Va.													(16)

b.	What is the output voltage of a step up chopper operating on a 220V DC supply with a pulse width of 150s and a blocking period of 40s.														(4)

(OR)

3.	Draw and explain the circuit diagram of buck-boost regulator and also derive the expression for the average output voltage, peak to peak ripple current, critical value of the inductor and the capacitor.

4.	a.	Draw and explain a typical control circuit of a single phase bidirectional controller with

resistive load.															(16)

b.	What are the disadvantages of unidirectional AC voltage controllers.			(4)

(OR)

5.	The single phase full wave controller supplies an RL load. The input rms voltage is

Vs = 120 V, 60 Hz. The load is such that h = 6.5mH and R = 2.5 . The delay angles of thyristors are equal. 1 = 2 =  /2. Determine i) the conductors angle of thyristor T1 (ii) the rms output voltage V0 (iii) the rms thyristor current IR (iv) the rms output current I0 (v) the average current of a thyristor IA (vi) the input power factor.

6.	Discuss the application of a voltage controller as a tap changer.

(OR)

7.	A single phase full wave ac voltage controller controls power flow from a 230V, 60 Hz, ac source into a resistive load. The maximum desired output power is 10 kW. Calculate

a.	the maximum rms current rating of thyristor IRM.

b.	the maximum average current rating of thyristor IAM.

c.	the peak current of thyristors Ip

d.	the peak value of thyristor voltage Vp

8.	With necessary waveforms, explain the operation of a center tapper transformer fed 1 to 1 cyclo converter with R load.

(OR)

9.	With relevant waveforms, explain the operation of 3 to 1  cyclo converter.