Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	GENERALISED THEORY OF ELECTRICAL MACHINES	       Time : 3 hours

Subject Code: 	EE338			         						                Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

Short note on the following machine’s Advantages, constructional diagram, operating principle and Torque equation

a.	Brush less DC motor               b.	Variable Reluctance motor                   		(10+10)

2.  	a.	Is phasor a time variant quantity? Explain                                           				(7)

b.	How can Differential equation for an AC circuit or machine be converted to a phasor equation?                                                           									(7)

c.	Prove that Active and Reactive powers in an Inductive circuit are given by

P = Re [  V       I      * ] and

Q = Imag [     V      I    * ]                                                   					(6)

(OR)

3. 	a.	Briefly explain the Transformer with a Movable secondary.            					(10)

b.	With a suitable diagram, derive the expression for transformer Voltage and

speed voltage.                                                             								(10)

4. 	a.	A 3 phase Induction motor has the following per phase parameters referred to stator:

Stator resistance : 0.30 Ohms

Rotor resistance : 0.45 Ohms

Stator and rotor leakage reactance : 2.1 Ohms

Magnetic reactance : 30 Ohms

Find out the parameters of an Equivalent 2 phase Induction motor if its per phase turns is same as that of the 3 phase Induction motor. 										(10)

b.	Briefly explain and derive the expression for Electric Torque           				(10)

(OR)

5. 	a.	What is the role of Invariance of Power in Linear Transformations?    				(6)

b.	With the help of suitable diagram, Explain Brush shifting transformation.                	(14)

6. 	a.	Enumerate the assumptions pertaining to the use of generalized mathematical model of DC

machines.                                                											(6)

b. 	A 15 kW, 240 V, 1500 rpm, 72 A DC shunt motor has an armature resistance of 0.3 Ohms. Neglect the effect of shunt field current.											(14)

i	This motor is to be operated under regenerative braking  conditions. If the armature current returned to the supply is 50 A, calculate the motor speed.

ii.	The motor is braked by plugging when working under rated conditions. For limiting the armature current to 1.2 times the rated current find:

[P.T.O]

1.	Resistance that must be inserted in the armature circuit

2.	Braking torque developed by motor

3.	Power input from the supply.

iii.	The motor is braked by dynamic braking when working under rated conditions. If the

armature current is to be limited to 1.2   times the rated armature current, find the resistance that must  be inserted in the armature circuit and the braking torque                    developed at the motor shaft.

iv.	Repeat part (iii), if the motor is initially running at 750 rpm  with rated armature

current. Neglect saturation.

(OR)

7. 	A  separately excited d.c. generator running at  4500 / π rpm, has the following parameters :

rf  = 80 Ω; Lf  = 40 H; ra  = 0.1 Ω;  La = 0.3 mH; Motional Inductance Md  = 0.8 H;

( or generated emf constant Kg  =  Md ώr  = 120 volts / field amp)

a.	The field is unexcited and the armature is open. Find the armature voltage as a function of time and sketch it, if a constant voltage of 160 volts is suddenly impressed across the field terminals.

b.	Sketch the rise of armature current in part (a), if the armature terminals  are initially short circuited.

c.	Armature voltage has attained steady value in part (a). Now  the armature is suddenly connected to a load of resistance 1.1 Ohm in the series with  an inductance  of 1.7 mH. Determine (i) armature current and     (ii) the armature terminal voltage as functions of time.

d.	In part ( c), find the electrical torque as a function of time. Obtain the mean value of torque also.

e.	In part ( c), after the armature current has become steady state, an additional   resistance of 0.4 Ω is inserted in the load circuit. Find the armature current, after this change.

8. 	a.	List out the Disturbances in synchronous machines and also list out the effect of rotor

oscillations on the performance of synchronous machines.                      				(8)

b.	A 50 kW, 400V, 3 phase, 4-pole, 50Hz wound rotor induction motor develops a maximum internal torque of 240% at a slip of 20% when working under rated voltage and frequency conditions and with slip rings short circuited.   									(12)

Determine :

i.	Full load slip

ii.	Rotor I 2 R loss at full load and

iii.	Starting torque in Nm.

Neglect stator resistance and rotational losses.

(OR)

9. 	a.	Develop the Basic Electromechanical equation for a synchronous machine

act as a motor.                                                                                                       		(14)

b.	A 3 phase, 4-pole induction motor has 48 stator and 52 rotor slots. Find the rotor speed at

which harmonic synchronous torque would be developed.          					(6)