Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	CONTROL SYSTEMS								              Time : 3 hours

Subject Code: 	EI203			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are components of feedback control systems?

2.	What is tachogenerator?

3.	What is steady state response?

4.	For the system with transfer function is given below, determine the type and order of the system G(S)H(S) = 10/S3 (S2 + 2S + 1)

5.	What is necessary conditions for stability?

6.	How the centroid is calculated in root locus technique?

7.	What is the corner frequency of the given transfer function, G(j) = 1/1+jT.

8.	What is the formula for the phase margin?

9.	Write the transfer function of PI controller.

10.	The union of two fuzzy sets A and B in u is a fuzzy set AUB, is defined as AUB = _______.

PART – B  (5 x 3 = 15 MARKS)

11.	Write the Mason’s gain formula?

12.	A unity feedback system has a open loop transfer function of

G(S) = 20 (S + 5)/ S2 (S + 0.1)(S + 3). Determine the steady state error for parabolic Input.

13.	Using Routh Criterion, determine the stability of the system represented by the characteristic equation, S4 + 8S3 + 18S2 + 16S + 5 = 0. Comment on the location of the roots of characteristic equation.

14.	What is Nichols chart?

15.	Draw the response of Fuzzy Logic Control Vs PID when the input is step.

PART – C  (5 x 15 = 75 MARKS)

16.	Consider the block diagram shown in fig. 1 Using block diagram reduction technique, obtain C/R.

G5

R

+

+

+

C

G1

G2

G3

G4

-

-

H2

H1

Fig.1

(OR)

[P.T.O]

17.	Consider the signal flow graph shown in fig. 2. Find X8/X1.

x5

G12

G8

G7

G13

x3

G6

G11

Fig. 2

G9

G5

G10

G1

x2

G2

x4

G3

x7

G4

x8

x1

18.	A system is given by differential equation, d2y/dt2 + 4dy/dt + 8y =8x, where y = output and x = input. Determine all time domain specifications for unit step input.

(OR)

19.	The open loop transfer function of a servo system with unity feedback is G(S) = 10/ S (0.1 S +1). Evaluate the static error constants of the system. Obtain the Steady state error of the system. When subjected to an input given by the polynomial r(t) = a0 + a1t +a2/2 t2.

20.	Determine the range of K for stability and ‘a’ so that the oscillates at frequency of 2 rad/sec of unity feedback system whose open loop transfer function is G(s)=K(s+1)/s3+as2+2s+1.

(OR)

21.	A unity feedback system has an open loop transfer function

G(s)=K(s+9)/s(s2+4s+11). Sketch the root locus.

22.	Sketch the Bode plot for the following transfer function and determine the system gain margin and phase margin.

G(s)=75 s(1+0.2s)/s(s2+16s+100)

(OR)

23.	The open loop transfer function of a system is G(S)H(S) = (1+4S)/S2(1+S)(1+2S). Determine the stability of closed loop system. If the closed loop system is not stable. Then find the number of poles lying on the right half of s – plane.

24.	Design the PID-controller with electronic version using op-amp.

(OR)

25.	Explain in detail fuzzy logic based temperature control systems.