Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	SIGNAL CONDITIONING CIRCUITS				              Time : 3 hours

Subject Code: 	EI235			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Ideal OP – Amp has _________input impedance.

2.	The frequency at which the magnitude of open loop transfer function is unity is called ______.

3.	The output of an integrator for square wave input is __________.

4.	When Vref is set to zero, the comparator acts as a _________.

5.	Instrumentation amplifiers have ________CMRR.

6.	Active filters eliminate the need of _________.

7.	7805 is a ________voltage regulator.

8.	During ________regulation, load current is held constant.

9.	The range of frequencies over which PLL can maintain the lock is called ___________.

10.	VCO stands for ___________.

PART – B  (5 x 3 = 15 MARKS)

11.	Define Input offset voltage.

12.	Draw an OP-Amp circuit for obtaining V0 = V1 – V2.

13.	Differentiate between Bessel, Chebyshev and Butterworth filters.

14.	What is Amplitude Modulation?

15.	List out the applications of PLL.

PART – C  (5 x 15 = 75 MARKS)

16.	Define the terms:

a.	CMRR		b.	PSRR		c.	Input offset current

d.	offset voltage	e.	slew rate

(OR)

17.	Explain in detail about Pole – zero and dominant pole compensation.

18.	Draw and explain working of a sample and Hold circuit.

(OR)

19.	Explain working of a stable mode of 555 timer and derive the expression for frequency.

20.	Design a fourth order Butterworth high pass filter having a lower cut off frequency of 1 kHz.

(OR)

21.	Explain in detail about switched capacitor filters.

22.	Draw and explain functional block diagram of 723 general purpose voltage regulator.

(OR)

23.	Explain working of a balanced modulator circuit.

24.	Explain working of a PLL with a neat block diagram.

(OR)

25.	a.	Derive an expression for output frequency of a VCO.

b.	How will you detect FM using PLL?