Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	LOGIC AND DISTRIBUTED CONTROL SYSTEMS	              Time : 3 hours

Subject Code: 	EI236			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	_________measures and records data very quickly and accurately.

2.	SCADA stands for _______________.

3.	_________converts bidirectional AC into pulsating unidirectional DC waveform in power supply unit of PLC.

4.	Electrical surges will not pass through __________in either direction.

5.	Good earthing in PLC will help to limit the effect of noise due to ____________.

6.	________involves turning the function on for one scan time at the trailing edge of a signal pattern.

7.	________is a human interface machine with monitor is a DCS.

8.	_________is a serial digital data transmission link connecting all the components  is a DCS system.

9.	In _____control mode an advanced digital controller is placed on the top of low level controller.

10.	_________is a device which interrupts the computer on a periodic basis.

PART – B  (5 x 3 = 15 MARKS)

11.	List out the characteristics of a Data logger.

12.	Distinguish between off line programming and on line programming in a PLC.

13.	Mention the advantages of PLC over hardwired control.

14.	What is a Field Bus?

15.	How will you integrate DCS system with PLC?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the working of SCADA with a neat block diagram.

(OR)

17.	Discuss various modes of digital control.

18.	a.	List out the general characteristics of registers is PLC.						(6)

b.	Write short notes on PLC counters with a suitable example.					(9)

(OR)

19.	a.	What are the step involved in construction of ladder diagram.					(7)

b.	Two conveyors feed a main conveyor. The count from each feeder conveyor is fed into a input register in PLC. Construct a PLC program to obtain the total number of parts in the main conveyor. If total count of main conveyor reaches 15, stop the convey.							(8)

20.	Write short notes on PLC Arithmetic functions and Number comparison functions.

(OR)

21.	a.	Write short notes on trouble shooting PLC malfunctions.						(6)

b.	Describe basic two axis Robot with PLC sequencer control.					(9)

22.	Draw the architectural block diagram of a DCS and explain the functions of each block.

(OR)

23.	Explain in detail about data highways.

24.	Discuss in detail about the various supervisory tasks and configurations of a DCS.

(OR)

25.	a.	Explain use of DCS in paper and pulp industry.								(8)

b.	Discuss the architecture of a HART protocol.								(7)