Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	MICROPROCESSORS AND MICROCONTROLLERS            Time : 3 hours

Subject Code: 	EI237			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is meant by program counter?

2.	What is the purpose of the 8085’s SID and SOD I/O lines?

3.	What is meant by an addressing mode?

4.	Via a certain groups of instructions, the sequential flow of a computer program can be altered.	What is the group?

5.	Which signals are used to activate an I/O port?

6.	What is the purpose of handshaking logic in a microprocessor I/O interface?

7	What is a non mask able interrupt?

8.	A 12 bit DAC has a full scale rating of 5.0 V. What is the 12 bit code that must be applied to this DAC to produce a 4.0V output?

9.	In the 8051, how many pins are designated as I/O port pins?

10.	What are the timers of 8051?

PART – B  (5 x 3 = 15 MARKS)

11.	What is the function of READY pin in a 8085 microprocessor?

12.	What is the value of the stack pointer after the following program run?

MVI	SP, 07FFH

PUSH	B

CALL SUBROUTINE

POP	B

ADD	B

PUSH	B

HALT

13.	What is meant by programmable I/O support device?

14.	List and brief three methods suitable for prioritizing the simultaneously interrupt request.

15.	Give the interrupt structure and vector address of 8051.

PART – C  (5 x 15 = 75 MARKS)

16.	Draw the pin out structure of 8085 microprocessor and explain the function of each pins

(OR)

17.	Draw timing diagram for an instruction MVI A 32H and explain the various stages of timing and labels associated with the structure.

18.	Name the different types/Groups of instruction set of 8085 microprocessor and explain them with suitable examples.

(OR)

[P.T.O]

19.	Write an 8085 assembly language program to add and subtract the contents of memory locations 0700H and 0701H and leave the sum and difference in memory locations 0702H and 0703H respectively.

20.	Draw a schematic of an 8085 interface 8bit memory-mapped input port and explain its design.

(OR)

21.	Draw the four possible I/O configurations of PORT A and PORT B, When 8255 is programmed for mode 1 operation and explain the configurations with its input and output timing structure.

22.	Draw a typical DMA controller architecture and explain about its functional parts.

(OR)

23.	Draw an interface diagram of DAC to an 8085 with the help of a design and explain the function of DAC conversion.

24.	Draw the functional blocks of internal architecture of 8051 microcontroller and explain each of the blocks.

(OR)

25.	Specify the features of 8051 serial modes and briefly explain each one of them with suitable diagram.