Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	ELECTRONIC CIRCUITS							              Time : 3 hours

Subject Code: 	EI238			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Transformer utility factor is given by --------------

2.	Ripple factor for half wave and full wave rectifier is given by ----------- and --------

3.	----------and ---------- are the two types of wave shaping circuits.

4.	Clamping circuits are used in -----------.

5.	Common mode rejection ratio (CMRR) is given by -------------.

6.	The frequency response from 50 Hz -20 kHz of RC coupled amplifier makes it useful in ---------.

7.	The effective resistance of a primary transformer of ratio 15:1 connected to an output load of 8Ω   is -----------

8.	If the feedback signal is in phase with the input signal then it is known as ----------

9.	------------ and ------------- are the two conditions that satisfy Barkhausen criterion.

10.	A crystal has L =0.33H, C=0.065pf, C2 =1.0 pf , R= 5.5 kΩ  , find the  Q factor ?

PART – B  (5 x 3 = 15 MARKS)

11. 	Write the comparison of Shunt and series Regulators.

12. 	Prove that the average (or d.c) value of the output of a differentiating circuit is zero.

13. 	Mention the advantages, disadvantages and applications of R.C coupled amplifier.

14. 	What are the types of feedback connections and what is their effect on input and output resistance.

15. 	Define Frequency stability of an oscillator? What are the factors that affect the change in oscillation Frequency ?

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the average values of output voltage and load current, ripple factor and efficiency of Half-wave and full wave rectifier.

(OR)

17.	Explain SMPS in detail.

18.	Explain linear wave shaping circuit.

(OR)

19. 	Give a detailed note about the positive and negative clipper and clamper circuits.

20.	It is the circuit diagram of darling ton amplifier determines (a) overall gain (b) ac emitter diode resistance for each transistor(c) total input resistance and (d) overall voltage gain.

(OR)

21. 	Explain the analysis of BJT amplifiers at low, medium and high frequencies.

22. 	Explain class B push pull amplifier and derive the expression for gain stability of feedback amplifier.

(OR)

23. 	In the above current series circuit, hfe=100,hie=2KΩ, Rc=470Ω, Re consists of two 100 transistor, one is bypassed, calculate the values gain, input resistance of the amplifier with and without feedback

24. 	Explain the operation and waveform of Schmitt trigger.

(OR)

25.	Explain Wein bridge oscillator and any one LC oscillator.