Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	SENSORS AND TRANSDUCERS					              Time : 3 hours

Subject Code: 	EI247			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The measurement of a quantity

a.	is an act of comparison of an unknown quantity with another quantity

b.	is an act of comparison of an unknown quantity with a known quantity whose accuracy may be known or may not be known

c.	is an act of comparison of an unknown quantity with a predefined acceptable standard which is accurately known

d.	none of the above

2.	Define Unit.

3.	Differentiate between Transducers and Inverse Transducers.

4.	While selecting a transducer for a particular application

a.	Only the input characteristics should be considered

b.	Only the output characteristics should be considered

c.	Only the transfer characteristics should be considered

d.	Input, output and transfer characteristics should be considered

5.	Draw the characteristics of positive temperature co–efficient thermistor.

6.	Define a zero–order instrument. Give an example.

7.   In liquid level systems, the capacitance of the system is:

a.	area			b.	volume			c.	level		d.	in-flowrate

8.  	Draw the equivalent circuit of Piezo-electric crystal.

9.  	The most important transducer for vibration, shock and general purpose absolute motion is

the_____________.

10.	A tachometer encoder can be used for measurement of displacement

a.	in both the directions

b.	in one direction only

c.	its output pulses are counted only for the forward direction

c.	its output pulses are counted only for the reverse direction

PART – B  (5 x 3 = 15 MARKS)

11.	A thermometer with time constant of 60 sec is used for measuring temperature cycling with 600sec time period. Find the time lag and ratio of output to input.

[P.T.O]

12.	Explain any three factors influencing the choice of transducers.

13.	Explain the sensitivity and linearity are two conflicting requirements in a resistance potential divider.

14.	Describe the different modes of operation  of piezo-electric transducer.Define and sketch binders and twisters..

15.	What are all the sub–systems of smart sensor?

PART – C  (5 x 15 = 75 MARKS)

16.	List the static characteristics of measurement system and explain.

(OR)

17.	Derive the impulse response of the first–order instrument.

18.	Describe the different criteria for selection of transducers for a particular application.

(OR)

19.	What are the different factors influencing the choice of transducer.

20.	Give the principle of operation, construction and applications of thermistors.

(OR)

21.	Explain the principle, characteristics and advantages of LVDT.

22.	Explain the principle of operation, equivalent circuit and applications of piezo–electric transducer

(OR)

23.	Explain how by using a differential arrangement, a capacitive transducer response  can be made linear.

24.	Explain the following.

a.	Shaft encoders		b.	Optical encoder		c.	magnetic encoders

(OR)

25.	Explain the following

a.	Sound sensors		b.	Vibration sensors		c.	Chemical sensors