Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	INSTRUMENTATION AND PROCESS CONTROL	              Time : 3 hours

Subject Code: 	EI249			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	L (eat) is equal to ________.

2.	______element is a control system gives the difference between reference signal and feed back signal.

3.	Servo mechanism is a _______control system.

4.	Integral control ________the order of the system.

5.	The terms in first column of Routh array are 6, 9, 2, 4, -3. The number of roots of characteristic equation is right half of S-plane is ___________.

6.	A system has negative feed back with two gain elements. ‘G’ is forward path gain and ‘H’ is feed back path gain. Then closed loop transfer function is __________.

7.	_________has negative temperature coefficient of resistance.

8.	LVDT stands for ___________.

9.	The principle of thermo couple is _________effect.

10.	U tube is used for measurement of ______________.

PART – B  (5 x 3 = 15 MARKS)

11.	Define the terms controller and control element.

12.	What is a control valve?

13.	State Masons gain formula.

14.	What is piezo resistive effect.

15.	State principle of a gas chromatograph.

PART – C  (5 x 15 = 75 MARKS)

16.	Draw a process control block diagram and explain various components present.

(OR)

17.	Find Laplace Transform of

a.	e-2t sin 2t		b	cos a t		c.	t e –t

18.	Explain construction and working of a pneumatic valve.

(OR)

19.	Discuss in detail about integral and derivative control.

20.	Obtain transfer function of the signal flow graph given below in fig. 1

1

G2

G3

G4

R

1

C

G1

-H1

-H2

[P.T.O]

(OR)

21.	Use Routh Hurwitz criterion to check whether the given characteristic equation is stable or not

s6 + 9s5 + 20s4 + 12s3 + 8s2 + 16s + 16 = 0.

22.	Explain working principle of a strain gauge and derive an expression for gauge factor.

(OR)

23.	Discuss construction and working of an RTD.

24.	Discuss the measurement of liquid level using radio active method and ultrasound method.

(OR)

25.	Discuss the working principle of a pH meter.