Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	ELECTRIC CIRCUIT ANALYSIS					              Time : 3 hours

Subject Code: 	EI250			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Draw the symbolic representation of current source.

2.	Write Kirchoffs voltage law.

3.	Define admittance.

4.	In 2 wattmeter method, the power factor is ________when one wattmeter shows zero reading.

5.	Write the expression for mutual inductance.

6.	Define node.

7.	The power delivered is maximum if the load impedance is equal to the supply circuit impedance. – True or False.

8.	Write 2 applications of maximum power transfer theorem.

9.	Define free response.

10.	Draw the equivalent circuit for inductor and capacitor at t = 0+ when there is no initial energy.

PART – B  (5 x 3 = 15 MARKS)

11.	Determine the equivalent resistance of the circuit of fig. 1 between A and B.

10

5

B

A

Fig.1

2

15

12.	A series RLC circuit with R = 10 , L = 1 mH and C = 1000 pF is connected across a sinusoidal source of 20V with variable frequency (a) compute the resonant frequency of the circuit (b) find the Q factor of the circuit at resonant frequency.

13.	Two coupled coils with respective self inductances L1 = 0.8 H and L2 0.2 H have a coupling coefficient of 0.6, coil 2 has 500 turns. If the current in coil 1 is i1 (t) = 10 sin 200 t determine the voltage at coil 2.

14.	State super position and Thevenin's theorem.

15.	A d.c voltage of 100V is applied to a series RL circuit with R = 25. What will be the current in the circuit at twice the time constant?

PART – C  (5 x 15 = 75 MARKS)

16.	Obtain the current through 3 resistor in the circuit shown below using Kirchoff’s laws.

28V

12

6

42V

3

(OR)

17.	Find the value of R and the current through it in the circuit shown below when the current in the branch OA is zero.

A

1

1.5

1

0

R

4

B

C

10V

2

18.	A 10 resistor and a 20 mH inductor are connected in series across a 250V, 60Hz supply. Find the impedance of the circuit, voltage across the resistor and inductor, power in the circuit.

(OR)

19.	Explain power measurement by two wattmeter method.

20.	In the network shown in Figure find V2 such that the current in (1 + j1) brand is zero. Use mesh current method.

3

6





(1+j1) 

j4

2

V2

V1 = 300V

(OR)

21.	Find the currents I1, I2 and I3 and the voltages Va and Vb in the network of figure by using nodal

analysis.

0.3

0.1

I1

I2

I3

+

0.2

30A

20A

+

va

vb

110V

-

120V

-

22.	Determine the current I in the network by using Thevenin's theorem.

A

I

4

6

10

10V

12V

B

(OR)

23.	For the circuit of the figure, find the value RL for maximum power delivered to it. Calculate also the maximum load power.

1.2

1.0

A

1.7V

12.5V

RL

1.6

0.4

1.4

1.4

B

24.	A series RLC circuit with R = 300, L = 1H and C = 100 x 10-6 F has constant voltage of 50V applied to it at t = 0. Find the maximum value of current. Assume zero initial condition.

(OR)

25.	In the circuit shown in figure below switches is in position 1 for a long time and brought to position 2 at time t = 0. Determine the circuit current.

50

1

2

0.2H

50V

100V

Find the energy dissipated in the resistors during transient.