Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	ELECTRON DEVICES								              Time : 3 hours

Subject Code: 	EI255			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the difference between a pentavalent atom and a trivalent atom?

2.	What properties of a semiconductor are determined from the Hall effect experiment?

3.	Define Diffusion capacitance.

4.   	Define dynamic resistance of a diode.

5.   	What are the consequences of early effect?

6.   	What is the need for biasing a transistor?

7.   	Draw the circuit of a common base transistor configuration.

8.  	What is the collector voltage if emitter base junction is open?

9. 	Define pinch off voltage of a FET,

10. 	What are the types of MOSFET?

PART – B  (5 x 3 = 15 MARKS)

11.	Distinguish between Drift current and Diffusion current.

12. 	What is meant by Avalanche breakdown ?

13. 	Write the Ebers-Moll equations for  and  relating the transistor currents to junction

voltages.

14. 	Draw the Hybrid   model for a transistor in common emitter configuration.

15 	How can an SCR be turned on and turned off ?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	With necessary diagrams explain conductors, insulators and semiconductors.

b.  	Describe Hall effect and derive the expression for Hall voltage.

(OR)

17.	a.  	Calculate the density of donor atoms which have to be added to intrinsic germanium to

produce n-type material of resistivity  and given .

b. 	Define Mobility and Conductivity and give its dimensions.

18.  	Derive the expression for the current flowing in a PN junction diode in terms of applied

potential.

(OR)

19.	a.  	Discuss about the temperature dependence of the VI characteristics of a PN diode.

b.  	Draw the circuit of a full wave rectifier and also derive the ripple factor.

20.	Derive the general expression for stability factor (S) and hence derive the expression for (S) for a

self-bias circuit.

(OR)

[P.T.O]

21.	Draw the circuit of a transistor in common emitter configuration and sketch the input and output characteristics. Explain how h-parameters can be determined from the characteristics.

22.	Draw the small signal equivalent circuit of common emitter amplifier and derive the expressions for voltage gain, current gain, input impedence and output impedence.

(OR)

23. 	Draw the small signal high frequency CE model of a transistor and explain the same. Compare common emitter, common base and common collector configurations.

24.	Explain the principle of operation of depletion MOSFET with necessary diagrams.

(OR)

25.	a. 	Distinguish between FET and BJT.

b. 	 Draw the equivalent circuit and characteristics of UJT and explain in detail.