Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	SIGNAL CONDITIONING CIRCUITS				              Time : 3 hours

Subject Code: 	EI258			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Draw the op-amp based voltage follower circuit.

2.	Define slew rate.

3.	Draw the positive clipper circuit.

4.	What is use of op-amp integrator and differentiator?

5.	Define sensitivity of strain gauge.

6.	What are the advantages of switched capacitor filter?

7.	What are different types of voltage regulators?

8.	Give examples for digital phase detectors.

9.	Define lock in range in PLL.

10.	Give some monolithic PLL ICs.

PART – B  (5 x 3 = 15 MARKS)

11.	What is common mode rejection ratio?

12.	Design an adder circuit using an op-amp to get the output expression as

Vo = -(0.1V1 + V2 +10V3) , where V1 , V2 ,V3   are the inputs.

13.	What is state variable filter?

14.	What are the advantages of switched Mode Power supply?

15.	How PLL is used in FM Demodulation?

PART – C  (5 x 15 = 75 MARKS)

16.	With a neat sketch and necessary equations, explain current mirror and widlar current source.

(OR)

17.	Explain different types of frequency compensation techniques.

18.	Draw and explain the Logarithmic amplifier with op-amp based circuitry.

(OR)

19.	Explain any two applications of 555 timer in monostable mode.

20.	With a neat sketch, explain the instrumentation amplifier using Transducer Bridge.

(OR)

21.	Explain with necessary equations high pass, band pass, and band rejection filter, transfer functions can be obtained using low pass transfer function.

22.	Explain the components of series op-amp regulator and explain the characteristics of three terminal IC regulators.

(OR)

23.	Write short notes on a) Current fold back b) Current boosting.

24.	Derive an expression for capture range of PLL.

(OR)

25.	Explain any two applications of PLL