Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	FOOD REFRIGERATION ENGINEERING AND TECHNOLOGY	 Time : 3 hours

Subject Code: 	FP222			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is co-efficient of performance of an refrigeration system?

2.	One ton of refrigeration is equal to ____________  removal of heat.

3.	How COP of heat pump is related to COP of refrigeration?

4.	What is the necessity of pre cooling of fruits and vegetables?

5.	What is the chilling injury of fruits and vegetables during the cold storage?

6.	The refrigerant code No for Ammonia is _______________

7.	Absolute humidity of air is expressed in ______________ Units.

8.	In modified atmosphere packaging of fruits gases normally used are____________

9.	What will be the range of the cryogenic temperature?

10.	Define Dew point temperature?

PART – B  (5 x 3 = 15 MARKS)

11.	Explain reverse Carnot cycle of refrigeration

12.	What is an vapour absorption system of Refrigeration.

13.	List out the important properties of ammonia refrigerant.

14.	List out the different types of Expansion valve systems.

15.	Define relative humidity, Dry bulb temp and wet bulb temp.

PART – C  (5 x 15 = 75 MARKS)

16.	What are the desirable properties of an ideal refrigerant and explain in details?

(OR)

17.	Explain with a neat schematic diagram of a simple saturated vapour compression refrigeration system.

18. 	Explain about the different types of compressors used in the refrigeration and cold storage units

(OR)

19.	A Ammonia refrigerator produces 30 tones of ice from 0ºC in a day. The temperature range of the working cycle in 25ºC to -15ºC, the vapour is dry saturated at the end of compression. Assume the actual COP is 60% of theoretical; calculate the HP required to drive the compressor. The properties of Ammonia at the above conditions are

| Temp | Liquid |  | Vapour |  |
| --- | --- | --- | --- | --- |
|  | Enthalpy kcal/kg (hf) | Enthalpy kcal/kgº K (sf) | Enthalpy kcal/kg (hg) | Enthalpy kcal/kgº K (Sg) |
| 25ºC | 23.91 | 0.083 | 315.3 | 1.072 |
| -15ºC | -13.04 | -0.051 | 311.9 | 1.209 |

[P.T.O]

20.	A spray cooling coil is chosen to operate under the following conditions

Air inlet condition 28ºC DBT and 21ºC WBT

Air outlet condition 10ºC DBT and 6ºC WBT

Total amount of air flow 2000m3/min

The chilled water inlet and outlet temperatures are 7ºC and 12ºC respectively.

Calculate:

a) The cooling load on the coil

b) Water flow rate through the coil.

(OR)

21.	How the humidification and dehumidification processes are carried out. Explain in details?

22	Briefly describe about hydro cooling with suitable examples.

(OR)

23.	Why the foods are frozened and briefly explain about a food freezing unit?

24.	What are the different types of Evaporative cooling systems available and explain any one of them?

(OR)

25.	Explain about the advantages and disadvantages of controlled atmosphere storage of fruits and vegetables.