Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	COMMUNICATION ENGINEERING				              Time : 3 hours

Subject Code: 	IT203			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the basic two types of Noise?

2.	Give anyone reasons why modulation is needed.

3.	What is Mode?

4.	Radiation angle for long wire antenna is approximately ___________

5.	What is the use of analog input filter?

6.	What is Parity bit?

7.	RS 232-Data send to phone lines rates up to ____________

8.	What is cell splitting?

9.	Formula of Signal power that reaches the receiver is  _____________

10.	What is EMI?

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the basic block diagram of super heterodyne receiver.

12.	A line with 75Ω impedance is terminated with 30Ω load. Find SWR?

13.	Give the frame format of one type of synchronous system.

14.	 What power is received when the transmitted signal is 1000W at 1 GHz (0.3m) the target

is 10,000m away (about 5.7 Miles), the target RCS is 1m2  and the antenna gain is40dB?

15.	Draw the block diagram of Ground Station in radar system.

PART – C  (5 x 15 = 75 MARKS)

16.	Write short notes about:													(8+7)

a.	External & Internal Noise		b.	Noise Figure

(OR)

17.	What is meant by suppressed carrier & Explain about single Side band Transmission system.

18.	Explain in details about waveguides, line & load matching.

(OR)

19.	Discuss briefly about various propagation modes with neat sketch.

20.	Describe briefly on Pulse code modulation with necessary diagrams.

(OR)

21.	Explain briefly about the QAM with its neat sketch.

22.	Explain briefly on RS 232 serial communication system.

(OR)

23.	With a block diagram, explain in detail about MODEM functions.

24.	Describe the following terms in Radar systems

a.	Radar cross section.(RCS)												(5)

b.	Radar range															(5)

c.	Various displays with neat sketch.     										(5)

(OR)

25.	Briefly explain about the fiber optic system characteristics and optical fibers with relevant diagrams.