Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	MICROPROCESSORS AND INTERFACING		               Time : 3 hours

Subject Code: 	IT217			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is meant by memory address space?

2. 	What is the function of data registers?

3. 	Distinguish between conditional and unconditional jump instruction.

4. 	How the 8088 processor differs from 8086 processor?

5. 	What is the use of bus cycle?

6. 	What is meant by bus arbitration?

7. 	Write the format of BSR control word in 8255.

8. 	Where the serial communication is preferred?

9. 	What is INTO?

10. 	List the different types of interrupts.

PART – B  (5 x 3 = 15 MARKS)

11. 	Mention the different types of flags available in the status register of 8088.

12. 	Distinguish between arithmetic and logical shift.

13. 	List the address bus status codes.

14. 	What is meant by DMA operation?

15. 	Mention different interrupt instructions.

PART – C  (5 x 15 = 75 MARKS)

16. 	Draw the internal architectural diagram of 8088 microprocessor and give the function of each unit.

(OR)

17. 	Explain how the stacks are handled in 8086microprocessor.

18. 	With a suitable example explain the different types of data transfer instructions used in 8088.

(OR)

19. 	Explain the different addressing modes used in 8088 microprocessor.

20. 	Explain the maximum mode of operation of 8086 microprocessor.

(OR)

21. 	With a neat sketch explain the isolated input / output interface.

22. 	Explain the architecture of 8253 and also explain how the control word formed in 8253.

(OR)

23. 	Explain the interfacing of keyboard and display with 8088.

24. With a neat sketch explain the operation of 8259A PIC.

(OR)

25. Explain the following:

a. Internal interrupt functions.		    b. Non maskable interrupt