Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	COMMUNICATION ENGINEERING				              Time : 3 hours

Subject Code: 	IT236			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	A signal suffers a 10% power reduction due to attenuation in the cable. The loss is ________dB.

2.	The modulated peak value of a signal is 10V and the unmodulated carrier value is 8V. What is the modulation index m?

3.	What is the maximum slew rate of a 10kHz sine wave of 2V amplitude

4.	If D is the dish diameter the dish gain is _________

5.	Companding is the process of _______ and  _________

6.	Determine even parity for the pattern 11001011

7.	Let the dialing number is 742 with 10 mark/break cycles/s and 0.5s between digits. What is the total dialing time?

8.	Echo time is______

9.	Mention any one advantage of using frequency in terms of GHz in satellite communication

10.	Say True/False: The uplink and downlink frequency and direction are different from each other.

PART – B  (5 x 3 = 15 MARKS)

11.	What is AGC? Why is it needed?

12.	Differentiate balanced and unbalanced line

13.	What is the difference between Synchronous system and Asynchronous system

14.	What is software handshaking? What are its benefits?

15.	What is RADAR cross section?

PART – C  (5 x 15 = 75 MARKS)

16.	What are the stages of superheterodyne reveiver? What is the role of each

(OR)

17.	Explain the working of Foster-Seeley detector with neat diagram

18.	Explain the principle of operation of parabolic dish antenna and mention its applications

(OR)

19.	What is coaxial cable? Compare it with twisted pair for Bandwidth and noise resistance.

20.	What is Eye pattern? What does it represent for a digital signal?

(OR)

21.	Write short notes on

a.	Accuracy			b.	Resolution			c.	Dynamic range

22.	How are DTMF tones recognized at the central office with filters and digital logic? How many filters are needed?

(OR)

23.	Discuss briefly the various solution to Echo

24.	Explain in detail the complete segment of GPS system

(OR)

25.	Explain about pulsed radar system? What are PRF and pulse width?