Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	MECHANICS OF MACHINES - II					              Time : 3 hours

Subject Code: 	ME208			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define D Alembert’s principle.

2.	What is static equilibrium

3.	Draw the turning moment diagram for a multi-cylinder engine

4.	What is the function of a flywheel of an engine

5.	What is Hammer – blow

6.	The balancing of rotating and reciprocating parts of an engine is necessary when it runs at ___________ speed.

7.	What is damped vibration

8.	What is period vibration

9.	What is the torsional stiffness of a shaft

10.	A shaft carrying two rotors at its ends will have ________ numbers of nodes

PART – B  (5 x 3 = 15 MARKS)

11.	Write short notes on crack effort.

12.	Define the terms coefficient of fluctuation of energy, work done per cycle and coefficient of fluctuation of speed.

13.	Explain the swaying couple

14.	Give the expression for the frequency and time period of simple and compound pendulum

15.	Derive an expressions for the torsionally equivalent shaft

PART – C  (5 x 15 = 75 MARKS)

16.	A connecting rod is suspended from a point 25mm above the center of small end and 650mm above its center of gravity, its mass being 37.5kg. When permitted to oscillate, the time period is found to be 1.87 seconds. Find the dynamical equivalent system constituted of two masses, one of which is located at the small end center.

(OR)

17.	A vertical petrol engine 100mm diameter and 120mm stroke has a connecting rod 250mm long. The mass of the piston is 1.1kg. The speed is 2000rpm. On the expansion stroke with a crank 20o from top dead center, the gas pressure is 700 kN/m2  determine.

a.	Net force on the piston

b.	Resultant load on the gudgeon pin and

c.	Thrust on the cylinder walls

18.	The turning moment diagram for a multi cylinder engine has been drawn to a vertical scale of 1mm = 650Nm and horizontal scale of 1mm = 4.5o. The areas above and below the mean torque line are -28, +380, -260, +310, -300, +242, -380, +265 and  -229mm2

[P.T.O]

The fluctuation of speed is limited to ±1.81 % of the mean speed which is 400 rpm. Density of

the rim materials is 7000 kg/m3 and width of the rim is 4.5 times its thickness. The hoop stress in the rim material is limited to 6N/mm2. Neglecting the effect of the boss and arms, determine the diameter and cross section of the flywheel rim.

(OR)

19.	A punching machine carries out 6 holes per minute. Each hole of 40mm diameter in 35mm thick plate requires 8 Nm of energy/mm2 of the sheared area. The punch has a stroke of 95mm. Find the power of the motor required if the mean speed of the flywheel is 20m/s. If total fluctuation of speed is not to exceed 3% of the mean speed, determine the mass of the flywheel.

20.	Three masses m1, m2, m3 are 4kg, 3kg and 2.5kg respectively. The corresponding radii of rotation 75mm, 85mm and 50mm respectively. The angular position of these masses are 45o, 135o and 240o measured in the counter clockwise direction from the +ve x- axis. Determine the amount of the counter mass at a radial distance of 75mm required for the static balance.

(OR)

21.	The following data relate to a single cylinder reciprocating engine:

Mass of the reciprocating parts = 40kg

Mass of revolving parts = 30 kg at 180mm radius

Speed = 150 rpm

Stroke = 350 mm

If 60% of the reciprocating parts and all the revolving parts are to be balanced, determine.

i.	The balance mass required at a radius of 320mm

ii.	The unbalanced force when the crank has turned 45o from the top dead center

22.	A machine mounted on spring and fitted with a dashpot has a mass of 60kg. There are three springs, each of stiffness 12N/mm. The amplitude of vibration reduces from 45 to 8 mm in two complete oscillations. Assuming that the damping force varies as the velocity, determine

i.	The damping coefficient

ii.	The radio of frequencies of damped and undamped vibrations and

iii.	The periodic time of damped vibration

(OR)

23.	The following data relate to a shaft held in long bearings.

Length of shaft = 1.2m

Diameter of shaft = 14mm

Mass of a rotor at mid point = 16kg

Modulus of elasticity of shaft material = 200 GN/m2

Eccentricity of center of mass of rotor from center of rotor = 0.4mm

Permissible stress in shaft material = 70 x 106 N/m2

Determine the critical speed of the shaft and the range of speed over which it is unsafe to run the shaft. Assume the shaft to be mass less.

24.	A shaft of 70mm diameter and 2.5m long carries two rotors A and C at its end a third rotor is placed at a distance of 1.5m from the left end (from rotor A), the mass moment of inertia of the rotors A, B and C are 0.15, 0.3 and 0.09 Kgm2. Find the natural frequency of the torsional vibration. The modulus of rigidity for the shaft material is 84 kN/mm2.

(OR)

25.	A reciprocating IC engine is coupled to a centrifugal pump through a pair of gears. The shaft from the flywheel of the engine to the gear wheel has a 48mm diameter and is 800mm long. The shaft from the pinion to the pump has a 32mm diameter and its 280mm long. Pump speed is four times the engine speed. Moments of inertia of flywheel, gear wheel, pinion and pump impeller are 1000 kg-m2, 14 kg-m2, 5 kg-m2 and 18 kg-m2 respectively. Find the natural frequency of the torsional oscillation of the system. G = 80 GN/m2.