Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	MACHINE DESIGN AND DRAWING				              Time : 3 hours

Subject Code: 	ME210			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is stress?

2.	What is stress concentration factor?

3.	Classify spring.

4.	Give any three materials used for springs.

5.	What is a shaft?

6.	What are the ‘2’ main type of couplings?

7.	The material for rivet is __________.

8.	Welding Joint is one of the ___________.

9.	Material used for crankshaft is __________.

10.	Flywheel stores _______________

PART – B  (5 x 3 = 15 MARKS)

11.	What are modes of failure?

12.	List out functions of spring.

13.	Classify couplings.

14.	Define coefficient of sliding.

15.	What are design parameters for piston.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain in detail the factors influencing Machine Design.

(OR)

17.	In an elastic material, principal stresses are tensile and compressive and the ratio being 4 : 1. Determining the limiting stress according to different theories of failure if tension test gives elastic limit of the material as 400 N/mm2. Assume Poisson’s ratio = 0.3.

18.	a.	Sketch a typical stress strain curve for ductile and brittle material and explain.		(10)

b.	What are Goodman and soderberg lines. 										(5)

(OR)

19.	Design a helical spring for a spring loaded safety valve from the following data.

Valve diameter 	=	65mm

Maximum pressure when the valve blows off freely	=	0.75 N/mm2

Valve lift when pressure rises from 0.7 to 0.75 N/mm = 3mm

Maximum permissible stress in the material of the spring = 500 N/mm2

Spring Index = 6

Modulus of rigidity of the spring material = 1 x 105 N/mm2

[P.T.O]

20.	In an axial flow rotary compressor, the shaft is subjected to maximum torque of 1500 Nm and a maximum bending moment of 3000 Nm. Neglecting the axial load on the compressor shaft, determine the diameter of the shaft. Assume that the load is applied gradually. The shear stress in the shaft is limited to 50 N/mm2.

Also design a hollow shaft for the above compressor taking inner diameter as 0.4 times outer diameter.

(OR)

21.	Design a bush type flexible flange coupling to transmit 15 H.P at 960 rpm. Allowable shear stress for the shaft and key may be taken as 53.5 N/mm2. The shear stress in the bolts should not exceed 35 N/mm2. The bearing pressure between bush and coupling should be limited to              2 N/mm2.

22.	A eccentrically loaded plate is welded to a frame as shown in fig. Design the welded joint if the tensile stress in the plate should not exceed 100 N/mm2 and that in weld 80 N/mm2.

ℓa

50

80

50 kN

80

ℓb

(OR)

23.	A knuckle joint is to transmit a force of 140,000N. Allowable stresses in tension, shear and compression are 75 N/mm2, 65 N/mm2 and 140 N/mm2 respectively. Design the Joint.

24.	Design a cast iron piston for single acting four stroke engine for the following specifications.

Cylinder bore = 100mm

Stroke	= 120mm

Maximum gas pressure = 0.65 N/mm2

Fuel consumption = 0.23 kg/kW/hr

Speed = 220 rpm.

(OR)

25. 	Design a flywheel for punching machine has a capacity of punching 30 holes of 20mm diameter per minute in a steel plate 16mm thick. The actual punching operation is estimated to last for a period of 36 degree of rotation of the punching machine crankshaft. The material of the plate has a ultimate shear stress of 350 N/mm2. This crankshaft is driven by flywheel shaft through a reduction gearing having a ratio 1 : 8. Assume that mechanical efficiency of the punching machine is 80%. Permissible coefficient of fluctuation of speed is 0.15.