Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	GAS DYNAMICS AND JET PROPULSION			              Time : 3 hours

Subject Code: 	ME217			         						                Maximum Marks: 100

(Use of gas dynamics table – permitted)

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write the steady flow energy equation for an adiabatic flow of air.

2.	Define “Mach Number”.

3.	Impulse function is defined as the sum of ___________ and  _________ .

4.	Stagnant velocity of sound (ao)  = ___________ .

5.	Define “ Fanno Flow”.

6.	In Rayleigh flow, the maximum enthalpy occurs at Mach Number = ________ .

7.	What is meant by strength of shock wave?

8.	Write Prandtl- Meyer relation.

9.	How will you classify propulsive engines?

10.	What are the types of liquid propellants used in rocket engines?

PART – B  (5 x 3 = 15 MARKS)

11.	What are the different regions of compressible flow?

12.	Write the important difference between Nozzle and Venturi.

13.	Differentiate between Isothermal flow and Fanno flow.

14.	Show a normal shock in h-s diagram with help of Rayleigh line and Fanno line.

15.	What are the types of rocket engines?

PART – C  (5 x 15 = 75 MARKS)

16.	The jet of a gas at 593 K ( = 1.3 & R = 469 J/kg K) has a Mach number of 1.2. Determine for local and stagnant conditions velocity of sound and enthalpy. Also determine the maximum attainable velocity of this jet.

(OR)

17.	The pressure, temperature and Mach number at the entry of a flow passage are 2.45bar, 26.5oC and 1.4 respectively. If the exit Mach number is 2.5 determine for adiabatic flow of a perfect gas( = 1.3 & R = 469 J/kg.K)  (i) Stagnant temperature, (ii) temperature and velocity of gas at exit and (iii)the flow rate per square meter of the inlet cross-section.

18.	A gas is isentropically expanded from P = 10 bar and T = 525oC in a nozzle to a pressure of      7.6 bar. If the rate of flow of the gas is 1.5 kg/s, determine (i) pressure, temperature and velocity at the nozzle throat and exit ; (ii) maximum possible velocity attainable by the gas  and (iii) the type of the nozzle and its throat area. Take  = 1.3 & R = 0.464 J/kg.K.

(OR)

19.	A conical diffuser has entry and exit diameter of 15 cm and 30 cm respectively. The pressure, temperature and velocity of air at entry are 0.69 bar, 340 K and 180 m/s respectively.  Determine (i) the exit pressure (ii) the velocity and (iii) the force exerted on the diffuser walls. Assume isentropic flow, Cp = 1.0 kJ/kg. K,  =1.4.

[P.T.O]

20.	A circular duct passes 8.25 kg/s of air at an exit Mach Number of 0.5. The entry pressure and temperature are 3.45 bar and 38o C respectively and the coefficient of friction 0.005.If the Mach Number at entry is 0.15, determine, (i) the diameter of the duct , (ii) length of the duct, (iii) pressure and temperature at the exit and (iv) stagnant pressure loss.

(OR)

21.	A combustion chamber in a gas turbine plant receives air at 350K, 0.55 bar and 75m/s. The air-fuel ratio is 29 and the calorific value of the fuel is 41.87MJ/kg. Taking   = 1.4 &                       R = 287 J/kg.K for the gas, determine : (i) the initial and final Mach numbers, (ii) final pressure, temperature and velocity of the gas , (iii) percent stagnation pressure loss in the combustion chamber, and (iv) the maximum stagnation temperature attainable.

22.	Starting from the energy equation for the flow through a normal shock, obtain the Prandtl-Meyer equation.

(OR)

23.	Air with Mach number 2.5 enters a convergent duct with an area ratio A2/A1 = 0.5. Under certain conditions, normal shock occurs at a test section where At/A1 = 0.6. For this condition, find exit Mach number and pressure ratio across the duct. (,A1, A2, At - Area at inlet, exit and test section respectively)

24.	The following data refers to a turbojet flying at an altitude of 9500m. Speed of turbojet is

800 kmph, propulsive efficiency = 55 %, overall is turbine = 17%. Drag on the plane = 6100 N, density = 0.17 kg/m3. Calorific value = 46,000 kJ / kg. Estimate the diameter of the jet, power output, A/F ratio and absolute velocity of the jet.

(OR)

25.	Explain the working principle of a Turbo- pump feed system used in a liquid propellant rocket.