Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	HEAT ENGINES AND FLUID MACHINERY		               Time : 3 hours

Subject Code: 	ME223			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Viscosity of a fluid.

2.	What is a submersible pump?

3.	Write down the impulse momentum equation.

4. 	A surge tank is ---------------------

5. 	Define “Thermodynamic system” and “Surroundings”.

6. 	State First Law of Thermodynamics.

7.	Draw the P-V diagram of Brayton cycle.

8.	Write the formula for Brake Power of an I C Engine.

9.	What is the use of Fins?

10.	Flow of fluid is considered turbulent if -------------

PART – B  (5 x 3 = 15 MARKS)

11.	Brief the phenomenon of Cavitation in Pumps.

12.	Write the various methods of Governing in Hydraulic Turbines.

13.	What is the relationship between a system and its environment when the system is

(a) adiabatic (b) isolated ?

14.	List out all the processes in a Dual cycle with a P-V Diagram.

15. 	What is a black body? Give its significance.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Define surface tension . What are the factors that affect surface tension?             			(3)

b.	A liquid drop of diameter 20mm has an internal pressure of  12.95 pascal. Find the

surface tension of the liquid film.                                                                           			(5)

c.	What is a manometer? Describe a U-tube manometer. The rigid limb of a U-tube

manometer containing mercury is opened to the atmosphere. The left limb contains

water and is connected to a pipe full of water at a pressure. The free surface of

mercury and the centre of the pipe are at the same level. If the difference between

the two mercury surfaces is 80 mm , find the pressure intensity of water in the pipe.		 (7)

(OR)

17.	a.	Explain with sketches the working of a single stage centrifugal pump.       				(5)

b.	The inlet and outlet blade angles of the impeller of a centrifugal pump are 600 and

400 respectively. The width of the impeller is 30mm. The inlet and oulet diameters of

the impeller are 180mm and 600mm respectively. Find the speed of the pump to

deliver 0.15m3/sec of water. Flow at inlet is entirely radial. Find also the head

generated by the impeller , manometric head and the impeller power. Take the

manometric efficiency of  the pump as 85%.                                                				(10)

18	a.	What is a turbine? What are the ways of classifying turbines?                   					(3)

b.	A Francis turbine with an overall efficiency of  75% is requires to produce 150Kw  at the

power shaft. It is working under a head (H) of  7.62 metre.The peripheral  velocity  is  0.262gHand the radial velocity is 0.962gH. The wheel runs at 150rpm and the hydraulic losses in the turbine are 22% of the available energy. Assuming radial discharge, determine

(i) the guide blade angle                     (ii) the wheel vane angle at inlet

(iii) diameter of the wheel at the inlet  		(iv) width of the wheel at inlet  				(12)

[P.T.O]

(OR)

19.	a.	What is a draft tube ? What are it’s functions? List out the various types.   					(3)

b.	A vertical  divergent draft tube  5.60 metres long is provided to a Francis turbine.  The diameters of the draft tube at the inlet and outlet are 500mm and 700mm respectively. The lower end of the tube is 1.4 metres below the tail race level. The velocity of water at the outlet of the tube is 1.25 m/sec. If the loss of energy head in the draft tube is 0.25 times the kinetic head at the outlet of the pipe , calculate

i.	absolute pressure head at the inlet of the draft tube

ii.	total energy heads at the inlet and outlet of the draft tube w.r.t tail water level.

iii.	power available at the inlet of the draft tube

iv.	efficiency of the draft tube.                              									(12)

20.	a.	 Prove that  for a gas  CP – CV  =  R                                                              				(3)

b.	1 Kg of gas at  1.1 bar , 270C  is compressed  to 6.6 bar as per the law  PV1.3 = C.

Calculate the work and heat transfer , if

i.	the gas is ethane (C2H6) with molar mass of 30 kg/k mol and CP  of  2.1 kJkgK.  		(6)

ii.	the gas is argon (Ar) with molar mass of 40 kg/k mol and CP  of  0.52 kJkgK.    		(6)

(OR)

21.	a.	What are the conditions for reversibility?                                                      				(2)

b.	Differentiate between heat pump and refrigerator.                                        				(3)

c.	A reversible engine operates between a source at 9720 C and two sinks, one at 1270 C and another at 270 C.  The energy rejected is same at both the sinks.  What   is the ratio of heat supplied to the heat rejected?  Also calculate the efficiency.  											(10)

22.	a.	Compare the  efficiency of Otto, Diesel and Dual cycles for the same  compression ratio and

heat rejection with the help of p-V and T-S diagrams.                                                   		 (5)

b.	In  an air standard Otto Cycle , the compression ratio is 7 and the  compression begins at  1 bar and 313 K. The heat added is 2510 kJ/kg.  Find 1. maximum  temperature and pressure of the cycle ;  2.  work done  per kg of air ; 3. cycle efficiency ; and 4. mean effective pressure.       Take for air, Cv = 0.713 kJ/kg K and R = 287 J/kg K.                             									(10)

(OR)

23	a.	What is the use of heat balance sheet of an engine?  Mention the various items to   be

determined to complete the heat balance sheet.                                 						(5)

b.	A four stroke petrol engine with a compression ratio of 6.5 to 1 and total   piston displacement of 5.2 x 10-3 m3 develops 100 kW brake power and consumes 33 kg of petrol per hour of calorific value

44300 kJ/kg at 3000 r.p.m Find: 1. Brake mean effective pressure ; 2. Brake thermal efficiency

At 1.013 bar and 150 C.  Take R for air 287 J/kg K.                             					(10)

24.	a.	Explain the different modes of heat transfer with appropriate expressions.  				(5)

b.	A composite wall consists of 10 cm thick layer of building brick, K=0.7 W/mK   and 3 cm

thick plaster, K = 0.5 W/mK0.  An insulating material of K = 0.08  W/mK  is to be added to reduce the heat transfer through the wall by 40%.  Find its thickness.                    				(10)

(OR)

25.	a.	Explain absorptivity, reflectivity and transmissivity.                                  					 (5)

b.	Two  large parallel planes at 800 K  and 600 K have emissivities of 0.5 and 0.8 respectively.  A radiation shield having an emmissivity of 0.1 on one side and emmissivity of 0.05 on the other side is placed between the plates.  Calculate the heat transfer rate by radiation per square meter with and without the radiation shield.   Comment on the results.         							(10)