Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	ENGINEERING THERMODYNAMICS				              Time : 3 hours

Subject Code: 	ME228			         						                Maximum Marks: 100

[Usage of Steam Tables and Compressibility chart may be permitted]

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is a Quasi-Static Process?

2.	Define an open System with an example?

3.	Define the Term COP.

4.	The Efficiency of the Carnot Engine will be Maximum, when the Initial Temperature is 0° K (True/False)

5.	What is a Triple Point?

6.	Write down the values of the Critical temperature and critical pressure of water.

7.	State Avogadro’s law.

8.	The value of Universal gas constant is ________________

9.	Define mean effective Pressure.

10.	The operations of Gas Turbines are based on _____________ cycle.

PART – B  (5 x 3 = 15 MARKS)

11.	Write down any five applications of First law of Thermodynamics in steady flow situations.

12.	State the Kelvin-plank statement of Second law of Thermodynamics.

13.	Define the dryness fraction of Steam.

14.	“The compressibility factor of an Ideal gas is Unity”- Justify.

15.	Define Air standard Efficiency.

PART – C  (5 x 15 = 75 MARKS)

16.	3Kg of gas at 75° C is heated at constant volume until its pressure is 2 times the initial pressure.

Determine (i) The Final temperature (ii) The heat transfer (iii) Change in internal energy,	iv)Change in Enthalpy and (v) Change in Entropy. Assume Cp=1.00 kJ/kgK and   Cv=0.071kJ/kg K.

(OR)

17.	At the entrance of a steady flow device, the pressure is 350 kpa and the specific volume is 0.04m3 /kg. At the out let, the pressure is 1 Mpa and specific volume is 0.02m3 /kg.  Find the flow work change in the process and comment on the result.

18.	Explain the theoretical Method of calculating the efficiency of the carnot cycle with the help of  PV and TS diagrams.

(OR)

19.	a)	An inventor claims that his proposed engine has the following specifications.

Power developed       		 :  50kW

Fuel burnt                  		 :   3kg/hr

[P.T.O]

Calorific value of the Fuel 	 :    75000 kJ/kg

Temperature limits   	             :    27° C and 627°C

Find out whether it is possible.

b.	Refrigerator removes heat from the refrigerated space of 2° C at the rate of 300 kJ/min and 	rejects it to the kitchen at 26° C at the rate of 345 kJ/min. Is it possible to operate the 	refrigerator?

20.	Find the Internal Energy of unit mass of steam at a pressure of 7 bar with the following conditions, by using STEAM TABLES only.

(a)   When its quality is 0.80

(b)   When it is dry and saturated.

(c)   When super heated, the degree of super heat being 65°C and the specific heat of super heated

steam at constant pressure is 2.1 kJ/kg K.

(OR)

21.	a.	Explain with a neat sketch the salient features of the Mollier diagram.

b.    Steam at 7 bar and 0.9 dryness fraction expands Isothermally to 1.5 bar.  Calculate by using

the steam Tables, the change in internal energy and change in Enthalpy.

22.	It is required to store 32kg of oxygen at 300 K and 10 Mpa.  The critical temperature and pressure for  Oxygen is 154.6 K and 5.04 Mpa.  Calculate the volume of the tank using Ideal gas equation and Generalized compressibility chart.

(OR)

23.	Explain the Method to determine the HCV of a liquid fuel by using Bomb Calorimeter.

24.	Air enters in to a constant volume cycle at 27° C and 1 bar and the compression ratio is 7.5. The maximum Temperature of the cycle is 1000 K.  Find the Air Standard efficiency and Mean effective pressure of the cycle.

(OR)

25.	Air enters in a constant pressure cycle at 100 kpa and 37° C. The compression ratio is 17:1. The

amount of heat supplied to the cycle is 450 kJ/kg.  Find the Air standard efficiency and Maximum temperature in the Cycle.