Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	THERMAL ENGINEERING						               Time : 3 hours

Subject Code: 	ME229			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The knocking tendency in petrol engine will increase when

a.	speed is increased		b.	speed is decreased

2.	A very sudden rise of pressure during combustion accompanied by metallic hammer like sound is called ________.

3.	For a steam nozzle, if P1 = inlet pressure, P2 = exit pressure and n is the index of isotropic expansion. The mass flow rate per unit area is maximum if

a.	P2P1≤2n+1nn+1			b.	P2P1≤2n+1nn−1

4.	The ratio of heat drop over moving blades to the total heat drop in the stage is ___________.

5.	With increase in clearance volume, the ideal work of compressing 1 kg if air _________.

6.	Rotary air compressors ________standard of compression.

7.	In a vapour compression refrigeration system, the lowest temperature during the cycle occurs after ___________.

8.	Hydrogen is used in Electrolux refrigeration system so as to __________the vapour pressure ammonia in evaporator.

9.	The relative humidity of air is defined as the ratio of ___________.

10.	RSHF = _____________.

PART – B  (5 x 3 = 15 MARKS)

11.	Comparison between a petrol engine and a diesel engine.

12.	In a steam nozzle, the team expands from 4 bar to 1 bar. The initial velocity is 60 m/s and the initial temperature is 200C. Determine the exit velocity if the nozzle efficiency is 92%.

13.	A single stage reciprocating compressor taxes 1m3 of air per minute at 1.013 bar and 15C and delivers it at 7 bar. Assuming that the law of compression is Pv1.35 = constant, and the clearance is negligible. Calculate the indicated power.

14.	Describe the mechanism of a simple vapour compression refrigeration system.

15.	For a sample of air having 22C DBT, relative humidity 30% at barometric pressure of 760mm of Hg. Calculate (a) Vapour pressure	(b) Humidity ratio		(c) Enthalpy

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Discuss the difference between theoretical and actual value timing diagrams of diesel

engine.																		(8)

b.	Explain with neat sketch of any one of the Lubrication system.						(7)

(OR)

17.	a.	Write a short notes on:														(8)

i.	knocking			ii.	Detonation

b.	Explain with neat sketch the water cooling system.								(7)

[P.T.O]

18.	Steam at a pressure of 15 bar and dryness fraction of 0.97 is discharged through a convergent – divergent nozzle to a back pressure of 0.2 bar. The mass flow rate is 9 kg/kwh. If the power developed is 220 kw, determine (a) Throat pressure (b) Number of nozzle required if each nozzle has a throat of rectangular cross section of 4mm x 8mm (c) If 12% of the overall isentropic enthalpy drop reheats by friction the steam in divergent portion find the cross-section of the exit rectangle.

(OR)

19.	A single stage impulse turbine rotor has a diameter of 1.2m running at 3000 rpm. The nozzle angle is 18. Blade speed ratio is 0.42. The ratio of relative velocity at outlet to relative velicity at inlet is 0.9. The outlet angle of blade is 3 smaller than the inlet angle. The steam flow rate is

5 kg/s. Draw the velocity diagram and find the following.

20.	A two-stage double acting air compressor, operating at 220 rpm takes in air at 1 bar and 27C/ The size of the L.P cylinder is 360 x 400mm, the stroke of H.P cylinder is the same as that of the L.P cylinder and the clearance of both the cylinders is 4%. The L.P cylinder discharges the air at a pressure of 4 bar. The air passes through the intercooler so that it enters the H.P cylinder at 27C and 3.80 bar, finally it is discharged from compressor at 15.2 bar. The value of ‘n’ in both the cylinder is 1.3, Cp = 1.0035 kJ/kg k and R = 0.287 kJ/kgk. Calculate.

a.	the heat rejected in the inter cooler.

b.	diameter of H.P cylinder.		c.	power required to drive H.P cylinder.

(OR)

21.	a.	Prove that the volumetric efficiency of a single stage compressor is given by		(10)

ηvol=1+k−kp2p11n, where k=vcvs

b.	What are the advantages of multistage compressor.								(5)

22.	Draw a neat diagram of lithium –bromide water absorption system and explain its working. List the major field of application of this system.

(OR)

23.	a.	Comparison between vapour compression and absorption system.					(10)

b.	The temperature limits of an ammonia refrigerating system are 25C and -10C. If the gas is

dry at the end of compression, calculate the coefficient of performance of the cycle. No undercooling of the liquid ammonia.											(5)

24.	Air at 10C dry bulb temperature and 90% relative humidity is to be heated and humidified to 35C dry bulb temperature and 22.5C wet bulb temperature. The air is pre-heated sensibly before passing to the air washer in which water is recirculated. The relative humidity of the air coming out of the air washer is 90%. This air is again reheated sensibly to obtain the final desired condition. Find

a.	the temperature to which the air should be preheated.

b.	the total heating required.		c.	the make up water required in the air washer.

(OR)

25.	A restaurant with a capacity of 100 persons is to be air-conditioned with the following conditions.

Outside conditions		:	30C DBT and 70% RH

Desired Inside conditions	:	23C DB T & 55% RH

Quantity of air supply		:	0.5 m3 / min / person

The desired conditions are achieved by cooling, dehumidifying and then heating. Determine

- Capacity of cooling coil in tonnes of refrigeration.

- Capacity of heating coil

- By-pass factor of the heating coil if its surface temperature is 35C