Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	HEAT AND MASS TRANSFER						              Time : 3 hours

Subject Code: 	ME230			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	If heat flow across a plane in a given slab should increase the temperature gradient at the plane should ________.

2.	In heat generation, surface convection also controls the _________temperature.

3.	For lumped parameter, model volume / Area ratio should be ___________.

4.	Most unsteady heat flow occurs

a.	through the walls of a furnace		b.	during annealing of castings

5.	For laminar conditions, the thickness of thermal boundary layer increases with its distance from the leading edge in proportion to

a.	x1/3				b.	x1/2

6.	The Nusselt number in natural heat transfer is a function of fluid Prandtl number and

a.	Reynolds number			b.	Grashoff number

7.	A heat exchanger with heat transfer surface area (A) and overall heat transfer coefficient (U) handles two fluids of heat capacities Cmax and Cmin, the parameter NTU (Number of Transfer Unit) used in the analysis of heat exchanger is specified as

a.	AUCmin			b.	A CminU

8.	The two types of mass transfer are __________ and _______.

9.	Milk spills over when it is boiled in an open vessel. The boiling of milk at this instant is referred to as ____________.

10.	The total emissive power E of a diffuse surface is related to readiation intensity I as, equal to

a.	 I		b.	4  I

PART – B  (5 x 3 = 15 MARKS)

11.	Consider an orange, assumed to be a sphere is producing an average internal heat generation (qg) during its ripening. Thermal conductivity of the material (k) and its centre temperature observed. Assuming one- dimensional, steady state conduction. Find out temperature distribution along the radius.

12.	An aluminum plate (ρ = 2707 kg/m3, CP = 0.896 kJ/kg 0C) and K = 200 W/mc of thickness 3 cm is at an initial, uniform temperature of 60C. Suddenly, it is subjected to uniform heat Flux q = 8000 W/m2, on one surface while the other surface is exposed to an air stream at 25C, with a heat transfer coefficient of h = 50 W/m2  0C.

a.	Is lumped system analysis to this case? If yes, what is the temperature of the plate?

Take ( τ   = 50 s)

13.	The velocity distribution in boundary layer is given by uU=yδ, where u is the velocity at a distance y from the plate and u = U at y =  being boundary layer thickness. Find

a.	the displacement thickness		b.	the momentum thickness

[P.T.O]

14.	A vessel contains a binary mixture of O2 and N2 with partial pressure in the ratio of 0.21 and 0.79 at 15C. The total pressure of the mixture is 1.1 bar. Calculate the

a.	molar concentrations		b.	mass densities

15.	A vertical plate of 350mm high and 420mm wide,  at 40C is exposed to saturated steam at

1 atm. Calculate the following.

a.	the film thickness at the bottom of the plate.

b.	the total heat flux to the plate.

PART – C  (5 x 15 = 75 MARKS)

16.	The inside temperature of a furnace wall, 200mm thick is 350C. The mean thermal conductivity of wall material is 1.35 W/mC. The heat transfer coefficient of outside is a function of temperature difference end is given by

h =  7.85 + 0.08T where T is the temperature difference between outside wall surface and surroundings. Determine the rate of heat transfer per unit area, if the surrounding temperature is 40C.

(OR)

17.	Find the steady state heat flux through the composite slab as shown in fig.1 and the interface temperature. The thermal conductivities of the two materials vary with temperature as given below.

KA = 0.05 [1 + 0.0065 t] W/m C

KB = 0.05 [1 | 0.0076 t] W /m C

Where temperature are in C

interface

600C

A

B

300C

1							3				Fig.1

LA = 50mm	LB = 100mm

18.	A carbon steel shaft of 0.2m diameter is heat treated in a gas – fired furnace whose gases are at 1200k and provide a convection coefficient of 80 W/m2k. If the shaft enters the furnace at 300K, how long must it remain in the Furnace to achieve a centre line temperature of 900 K?

Take properties of carbon steel	:	 ρ = 7854 / kg/m3,

K = 48.8 W/mK, CP = 559 J/kg k

(OR)

19.	An apple, which can be considered as s sphere of 8cm diameter, is initially at a uniform temperature of 25C. It is put into a freezer at -15C and the heat transfer co-efficient between the surface of the apple and the surroundings in the freezer is 15 W/m2  0C. If the thermo physical properties of apple are given to be : ρ = 840 kg/m3, CP = 3.6 kJ/kg0C, k = 0.513 W/m 0C and      = 1.3 x 10 -7 m2/s. Determine.

a.	Centre temperature of the apple after 1 hour.

b.	Surface temperature of apple at that time, and

c.	Amount of heat transferred from the apple.

20.	A Fine wire of 0.2m diameter is maintained at a constant temperature of 64C by an electric current. The wire is exposed to air at 1 bar and 10C. Calculate the electric power necessary to maintain the wire temperature if the length of wire is 1m.

(OR)

21.	Air at 30C and at atmospheric pressure flows at a velocity of 2.2 m/s over a plate maintained at 90C. The length and the width of the plate are 900mm and 450mm respectively. Calculate the heat transfer rate from

a.	First half of the plate		b.	Full plate		c.	next half of the plate.

22.	In a double pipe heat exchanger mhCPh = 0.5 mcCpc. The inlet temperature of hot and cold fluids is th1 and tc1. Deduce an expression in terms of th1 and tc1, th2 for the ratio of the area of the counter flow heat exchanger to that of parallel flow heat exchanger which will give the same hot fluid outlet temperature th2. Find the ratio if th1= 150C, tc1 = 30C and th2 = 90C.

(OR)

23.	In a Stefan tube experiment with corbon tetra chloride and oxygen, the following data are noted:

Diameter of the tube = 10 mm

Length of  tube above liquid surface = 150mm

Temperature maintained = 0C

Pressure maintained = 760mm Hg

Vapour pressure of CCI4 at 0C = 33mm Hg

Evaporation of CCI4 = 0.03g

Time of evaporation = 10 hours

Estimate the diffusion coefficient of carbon tetra chloride into air.

24.	Consider two parallel plates, one at 1000k with emissivity 0.8 and other is at 300 k having emissivity 0.6.. A radiation shield is placed between them. The shield emissivity as 0.1 on the side facing hot plate and 0.3 on the side facing cold plate. Calculate percentage reduction in radiation heat transfer as a result of radiation shield.

(OR)

25.	Water at atmospheric pressure is to be boiled in polished copper pan. The diameter of the pan is 350mm and is kept at 115C. Calculate the following.

a.	Power of the burner		b.	Rate of evaporation

c.	Critical heat flux for these conditions.